Table of Contents

# NEXA-web, Handover and Maintenance Guide

Version: 3.4 Guide date: 2026-09-11 Release described: NEXA V1, model and data version 1.0.0 Live at: https://carolinehvermette.github.io/NEXA-Web/, the permanent address since 2026-09-22 (first published 2026-08-24) Repositories: github.com/CarolineHVermette/NEXA-Web, which serves the site, and github.com/orcunkoraliseri/NEXA-web, the predecessor, which mirrors the code and serves nothing Prepared by: Orcun Koral Iseri, Postdoc, Resilient Habitat Lab, Concordia University Prepared for: Dr. Caroline Hachem-Vermette / RHLab

## How to read this guide

Two files carry the same text. Read the .html, which is the easier one on screen. Comment in the .docx, which is the one Word can mark up and track changes in. Send the marked-up .docx back and your comments become the next revision.

This guide contains no passwords, tokens or API keys. It records which accounts and permissions are required, not their secrets.

### Structure

| If you are | Read | About |
| --- | --- | --- |
| Going to use or present the tool | Part I | 20 minutes |
| Going to change the tool | Part I, then Part II | 40 minutes |
| Going to own, run and maintain it | Part III, with Parts I and II as orientation | It is a reference, not a read |

Part I answers what is this and what does the number mean. Part II answers where do I start and how do I change something. Part III is the reference, and it is the part that lets RHLab run the tool without the author. Parts I and II summarise; where they and Part III disagree, Part III is current.

### Where to start

| If you want | Read |
| --- | --- |
| The one thing that matters most for the handover | Section 6.0, where the numbers actually come from |
| Whether a number on screen can be traced | Section 5, the results data map, and 5.1, the floor area rule |
| What the tool cannot do | Part I, section G, and section 12.3 |
| How to run it without the author | Part II, then sections 8, 10 and 11 |
| How to add new information to the tool | Section 8, which opens with a table routing every kind of change to its procedure and its files |

### Relationship to the public methodology

The tool’s own documentation.html explains the science to a stakeholder. This guide explains the system to a maintainer. The separation was an explicit instruction, given twice: internal material must not leak into the public methodology, and this guide must not be reduced to a pointer at it. All three of the documents merged into this one were internal, so merging them does not touch that boundary.

The revision history of this guide is Appendix D.

# Part I. The tool and its results

For a reader who will use or present the tool. No code, no procedures. The same material in the form the public sees it is the tool’s own documentation.html page.

## A. The NEXA-web tool

NEXA stands for Neighbourhood Energy eXploration & Analysis. It is an interactive neighbourhood energy explorer. You compose a neighbourhood by typology, then overlay energy demand and generation systems, electric mobility and green infrastructure, and the tool shows what each choice does to a small set of energy indicators.

The important idea is what sits behind the choices. Energy and solar results for every neighbourhood in the tool come from completed EnergyPlus simulation campaigns, across a ladder of efficiency scenarios and across seven climates, of which five are published today. They are never simulated on demand; the indicators built from them (the ratio of performance, absolute totals, net grid demand, landscape solar and the V2G chain) are calculated in the page from those results and stated constants (see section 5). That is why results appear instantly, and why the tool answers for the archetypes and climates that were simulated.

A dozen choices in front, 2,974 simulated results behind.

### Intended use

NEXA-web is an early stage comparison tool, for a planner, a municipality or a researcher who wants to see how neighbourhood form, envelope standard, on site generation and electric mobility move a small set of energy indicators, before any design work begins.

It is not, in this version, a design or compliance tool for an individual building; not an optimisation engine — no formal optimisation method is implemented, so the tool never ranks the scenarios and never recommends one; and not a substitute for a project specific energy model.

The distinction matters when the tool is presented. NEXA-web answers “how do these options compare to one another”, not “what should be built here”.

## B. The four layers

A session is a sequence of choices, each one narrowing what comes next. The sidebar keeps every earlier choice visible, so the design under discussion is always on screen.

| Layer | You choose | The tool answers |
| --- | --- | --- |
| 1. Neighbourhood | Land use, context, density, layout, and the climate and envelope code | The matching neighbourhood units, their composition, floor area, and a 3D model of each |
| 2. Energy | The efficiency scenario, the heating and hot water systems, and which generation technologies to install | Annual energy use intensity, the end use breakdown as a treemap, and the rooftop solar result |
| 3. Mobility (preliminary) | Electric vehicle uptake, public transport, charging stations, vehicle to grid | Daily charging demand, charger counts, and the vehicle to grid energy available |
| 4. Green infrastructure (preliminary) | Green roofs, vertical greening, green spaces, urban agriculture, landscape solar (selection and visualisation across eight options, with one quantified option) | Selection and visualisation; landscape solar generation (the single quantified option) |
| Summary | Nothing, it consolidates | The whole design on one dashboard, with the net energy balance |

The layers are deliberately independent: a mobility choice can be revised without losing the energy scenario. Combinations that have no simulated answer are disabled as you choose, so an unanswerable design cannot be assembled.

## C. Basis of the results

### The 35 neighbourhood units

A neighbourhood unit is a fixed composition of buildings, not a single building. There are 35.

| Family | Code | Count | What they are |
| --- | --- | --- | --- |
| Compact residential | RC-* | 10 | Detached, townhouse, mid rise and high rise research sets |
| Residential | RS-* | 5 | Single family, semi detached, townhouse, low and mid rise apartment |
| Mixed use | MU-* | 9 | Commercial ground floor with housing above, main street retail, mixed high rise |
| Commercial and civic | CC-* | 9 | Office parks, retail, supermarkets, schools, outpatient healthcare, food service |
| Industrial and data centre | IC-* | 2 | Large data centres, edge computing, light industrial |

The buildings inside them are not invented. They begin from internationally recognised reference prototypes — sixteen ASHRAE 90.1-2022 commercial prototypes, three IECC 2024 residential variants, two data centre archetypes and seven supplementary buildings — and are then transformed to Canadian practice: NECB 2017 for commercial and institutional buildings, NBC 9.36 for housing, with Canadian occupancy and plug load schedules from NRCan’s BTAP data. The full account is Part III, section 6.

### The five climate arms

Seven climate arms were simulated. Five are published in NEXA V1. The ASHRAE arm is a United States reference case and Zone 8, Chisasibi, is held back at the supervisor’s request; both remain in the research database.

Each unit is simulated in five Canadian NECB 2017 climate zones, each carried by a real city and a real weather file. Montreal additionally carries a 1983 Quebec reference envelope arm, for reasoning about the existing stock rather than about new construction. 154 neighbourhood and climate pairs are published.

| Option | Zone | Representative city |
| --- | --- | --- |
| NECB Zone 4 | NECB 4 | Vancouver |
| NECB Zone 5 | NECB 5 | Toronto |
| NECB Zone 6 | NECB 6 | Montreal |
| NECB Zone 7A | NECB 7A | Winnipeg |
| NECB Zone 7B | NECB 7B | Fort McMurray |

Two arms were withdrawn from the public tool, and neither was deleted. The ASHRAE 90.1 arm on Buffalo NY was withdrawn on 2026-08-17: it is a United States reference case on a United States weather file, not a Canadian climate. NECB Zone 8, Chisasibi, was withdrawn on 2026-08-24 on Dr. Hachem-Vermette’s instruction. Both remain in the research database and in Part III; only their quantitative results are not exposed to the public tool.

Both withdrawals were confirmed as V1 decisions on 2026-09-09, and each carries a note about what comes after. Buffalo, her decision E8: it is kept as a research and reference arm, it is not presented publicly as a Canadian climate, and ideally it is re-run later under Canadian weather rather than retired. Chisasibi, her decision E7: it stays in the research database and the technical documentation, and no quantitative public result is shown for Zone 8 in V1.

“Vintage” is no longer used in the interface. The Montreal arm reads 1983 Quebec reference envelope, which is what it is.

### The five scenarios, applied cumulatively

This is the single most misread part of the tool. Each rung contains everything below it. Scenario 3 is not “hot water”; it is envelope plus heat pump plus hot water. That is why each label names the whole package.

The names on screen are the short ones Dr. Hachem-Vermette asked for on 2026-08-24, with a terminology legend wherever the abbreviations first appear. HPerf is a high performance envelope, Space HP a space heating heat pump, HPWH a heat pump water heater and EEM the remaining energy efficiency measures.

| Rung | On screen | What it actually is |
| --- | --- | --- |
| Baseline | Baseline | Code compliant reference case built to NECB 2017 as designed. Native PV only |
| 1 | HPerf | High performance walls and roof, triple glazing, foundation insulation, air leakage cut to a quarter |
| 2 | HPerf + Space HP | Adds a cold climate air source heat pump, inverter cooling, heat recovery |
| 3 | HPerf + Space HP + HPWH | Adds a transcritical CO2 heat pump water heater with a stratified tank |
| 4 | HPerf + Space HP + HPWH + EEM | Adds automated shading and daylight dimming, LED lighting, occupancy trim, ENERGY STAR plug loads, and electrification of gas appliances in housing |

The Baseline rung is a code-compliant reference case, a new neighbourhood built to NECB 2017. It is not a survey of existing buildings and it is not the current condition of any real neighbourhood. The 1983 Quebec reference envelope (Montreal only) is the closest thing the tool has to an earlier condition; it is an envelope built to the 1983 Quebec construction requirements (Regulation E-1.1, r. 1, Order in Council 89-83), representing a code-era reference rather than a survey of existing stock, and is compared against itself.

A sixth option, ideal thermal load, strips out the heating and cooling equipment to expose the pure thermal demand of the envelope. It is for district energy sizing, and it cannot be compared against the other rungs, because it is not delivered energy.

## D. The two headline indicators

### Energy use intensity

Energy is reported as site energy: electricity plus gas as metered, with no source energy multipliers and no carbon factors applied.

Intensity is per square metre of heated and cooled floor area, which is the area the simulation actually conditions. The floor area printed on the neighbourhood table is the total built area, which includes unheated attics and basements. On the house based neighbourhoods the total is about twice the heated and cooled area. The two are different numbers on purpose, and the page says which is which, because dividing by the wrong one changes the answer by a factor of two. The rule a maintainer must follow is Part III, section 5.1.

### Ratio of Performance

In plain words: what share of the energy this neighbourhood uses in a year can its own rooftop solar produce in that same year?

RoP = annual PV generation (kWh/m2) / annual energy demand (kWh/m2)

Both quantities are annual and both use the same floor area, so the area cancels and RoP is a pure ratio. RoP of 1.00 means the roof generates over a year as much as the neighbourhood consumes. RoP of 0.35 means it covers 35 per cent.

A worked example, detached houses (RC-D) under NECB 2017. At scenario 1 the roof produces 100.4 kWh/m2 per year against a demand of 107.0, so RoP is 0.94. At scenario 4 the roof is unchanged but demand has fallen to 60.6, so RoP is 1.66. This is the characteristic shape of the result: RoP rises because demand falls, not because panels were added.

Two things RoP does not say. It is an annual balance, not an hourly one: a neighbourhood at RoP 1.00 still imports electricity on a December evening and exports at midday in June, so annual net zero is not grid independence, and RoP says nothing about storage or peak demand. And it is a ratio of two intensities, so a low RoP is often a statement about building form rather than about the solar design — a high rise has roughly the same roof as a low rise but many times the floor area beneath it, so its RoP is structurally lower with an identical array.

## E. Rooftop solar

The solar assumptions belong to the roof, not to the neighbourhood, and there are two roof groups because the two carry genuinely different arrays.

|  | Flat roof | Pitched roof |
| --- | --- | --- |
| Neighbourhoods | 31 of 35 | 4 of 35: RC-R, RC-D, RC-ML, RC-T |
| Array | A single south facing rack at 45 degrees tilt | Flush on the south facing pitch, tilt equals the roof pitch |
| Panel efficiency | 23 per cent | 23 per cent |
| Active fraction of the reference surface | 1.0 on the rack | 0.85 of the south roof face |
| Ground coverage ratio | 0.40 | Does not apply, a flush array has no row spacing |

Common to both: 14 per cent system losses from DC to AC, 96 per cent inverter efficiency, arrays oriented to true south. The model itself is Part III, section 6.6; every constant is in the assumptions register, Part III, section 15.2.

A correction, made on 2026-08-24, and it was ours. Earlier versions of this guide, and the tool itself, named 18.65 per cent as the module efficiency behind the published results. That was wrong, and Dr. Hachem-Vermette was right not to let the question close. Every published PV number was produced at 23 per cent panel efficiency. The figure 18.65 per cent is a cell efficiency from a retired calculation track, behind an active area fraction of 0.80; the 20 per cent that also circulated is a module efficiency from a literature reference, behind a 0.90 fraction. Comparing them without their fractions compares a cell with a module, which is why the difference never closed. The generated energy is unaffected, because it is simulation output from the 23 per cent model. What was wrong was the assumption printed beside it. No run was repeated and no stored value moved.

Total PV array area, in square metres, is now a headline figure, taken from the photovoltaic surfaces of the simulation model itself, and shown for 26 of the 35 neighbourhoods. For the other nine the row is absent rather than estimated: their stored array does not reproduce the generation the tool publishes, and an area that does not reproduce the generation is not the area behind it. Generation intensity in kWh per square metre of floor area is no longer a headline figure, because a generation per square metre of floor invites being read as a figure per square metre of array.

Why solar output per square metre of floor varies so much between neighbourhoods. The numerator is limited by roof area, while the denominator grows with every storey. Low rise forms have a lot of roof per square metre of floor, and so a high intensity; tall forms spread one roof over many storeys, and so a low one. Detached houses land near 100 kWh/m2, mid rise near 66 to 75, high rise near 46 and, for the tallest, near 26. That spread is physics, not an inconsistency in the tool.

## F. Maturity of a figure

Four words, and no others, describe the standing of any figure in the tool. They appear on the pages themselves, not only in this guide.

| Term | Meaning |
| --- | --- |
| Simulation-backed | The value comes from an EnergyPlus run |
| Preliminary | The value comes from a documented calculation chain, not from a simulation |
| In development | Being worked on, no figure is shown |
| Not modelled yet | No quantitative method exists. The option is visible, disabled, and the reason is given |

Maturity is one axis. Provenance is a second, and the tool carries both. Beside every result that has one, the pages print where the number came from, in two words only: Directly simulated, meaning it comes straight out of an EnergyPlus run, or Derived from simulation, meaning a calculation was applied to values that did. The mapping, result by result, is LMN_CONFIG.provenance. results in js/config.js, and it is the mapping that decides the label, not the page. A figure therefore carries a maturity term and, where applicable, a provenance label: the rooftop solar total is Simulation-backed and Derived from simulation at once, because it is a simulated intensity multiplied by a simulated area.

One inconsistency, and it is CHV’s to settle. Her instruction of 2026-08-24 added “Derived from simulation” to the published list of status terms by name, so LMN_CONFIG.statusTerms now holds five terms and the Scenario Summary page prints all five in one line. Her closeout checklist of 2026-09-09, item A20, asks that maturity and provenance be kept separate. Both cannot hold at once. The tool today follows the earlier instruction. Splitting the printed list back to four maturity terms, with the two provenance labels shown as their own legend, is one edit in js/config.js and one in js/finish-design.js, and it waits on her word.

In practice:

- Building energy, rooftop solar and the efficiency scenarios are simulation backed. These are the results the tool exists to deliver.

- Electric vehicle and vehicle to grid figures are preliminary. They come from households times vehicles per household times a daily charging demand.

- Landscape solar is preliminary. It is a land area assumption multiplied by a specific yield. The same land allocation is applied to every neighbourhood by decision, so that figure does not vary between them.

- Facade solar is restricted. It was tested on three tall building archetypes in Montreal only, is offered only where one of those appears, is marked preliminary, and is excluded from the totals.

- Biomass, wind, geothermal, solar thermal and most green infrastructure options are visible and disabled, because no quantitative model exists for them in this round.

## G. Limitations

Everything here is also on the tool’s own documentation page. It is repeated because a reader who is shown the tool should hear it from the person showing it, not discover it later from a number.

In the building simulation. Duct leakage is not modelled: the detailed airflow solver was switched off to avoid numerical failures at neighbourhood scale, which is worth about 5 per cent of energy use intensity and a tenfold saving in run time. Daylight controls and automated blinds are skipped where they are impractical or where the engine could not carry them. In the stratified hot water tank, backup resistance energy is reported by EnergyPlus under space heating rather than water heating: the total is conserved, the split between those two columns is not exact. Data centres and laboratories barely respond to envelope measures, because their energy is dominated by process loads that no building level measure touches.

In the solar modelling. Snow is not modelled at all, so winter yield in zones 7A, 7B and 8 is optimistic by roughly 10 to 25 per cent, and the results page says so in those zones. The 45 degree rack tilt is energy optimal for high latitudes but steeper than the 10 to 15 degrees that wind uplift and ballast limits normally allow on a commercial flat roof. There is no battery, no peak shaving and no grid feed in logic: generated electricity simply meets the building’s load.

In coverage. Five compact residential units outside Montreal rest on a United States prototype rather than the Canadian baseline, and are not published pending re-simulation. One mixed use case in zone 7B has no baseline upstream, and the tool declines the combination and says why, rather than filling the hole with a neighbouring value. That principle holds throughout: where the tool has no answer it says so, and never substitutes.

In the mobility and green layers. On site solar is not netted against the electric vehicle load, so the mobility indicator can only report a deficit. Two efficiency assumptions in the vehicle to grid chain are still open with the supervisor. They are stated with their consequences in Part III, section 15.3. In green infrastructure, Layer 4 provides selection and visualisation across eight options, with one quantified option (landscape PV, a fixed calculation chain). The other seven options are labelled Not modelled yet.

# Part II. Developer orientation

For a developer or researcher arriving on the project who needs to find their way around, change something and publish it, without asking the original author. Every path here is a path inside the repository, not a path on anyone’s machine. Each section names the place in Part III where the same subject is stated in full.

## H. The system in one paragraph

NEXA-web is a static site. No server, no database, no build step, no dependency install. Clone it, serve the folder over HTTP, and it runs. Every number it displays was computed offline and shipped inside js/data.js; the website calculates nothing scientific of its own. Offline means one of three things, and the distinction matters more than any other in this guide:

|  | Kind | What produced it | Examples |
| --- | --- | --- | --- |
| 1 | Simulation output | An EnergyPlus campaign, run upstream in idf_reader | Building energy use by end use, rooftop solar generation |
| 2 | Derived from simulation | A calculation applied to those outputs, upstream or in the page, from stated constants | Energy use intensity, ratio of performance, net grid demand |
| 3 | Preliminary calculation chain | A separate offline calculation that is not a simulation at all | Mobility and vehicle to grid (section 6.8), landscape solar (section 6.9) |

Every constant that appears on more than one page lives in js/config.js with a comment saying where it came from. Publishing is a git push: GitHub Pages serves the repository as it stands.

The consequence, and it is the single most important thing to understand: you cannot change a result by editing the website. A wrong number means one of three things: a wrong value in js/data.js, which is fixed by re importing from the simulation campaign; a wrong simulation, which is fixed upstream and then re imported; or a wrong input to one of the Preliminary calculation chains, which is fixed in that chain and the result regenerated. Which of the three it is, is the first question to answer, and section 15.0 is how to answer it.

Modularity: The architecture and documentation remain modular so that future RHLab researchers can maintain and extend NEXA-web: one page per layer; one script per page in js/; one source of truth in js/config.js; js/data.js generated and never hand edited; the persistent sidebar generated once by buildSidebar() in js/sidebar.js; and a new layer added by following the section 8.9 procedure without editing the existing layers. The two exceptions where this is not true today are recorded honestly: ENERGY_COLORS moved to js/config.js on 2026-09-09, which closes the first of them; and js/data.js carries a dead "MU-S" breakdown key for a neighbourhood archetype that does not exist in the published set, found and recorded on 2026-08-27, left in place deliberately because no code path reads it and documented here so a future maintainer does not spend time investigating what a neighbourhood called MU-S was.

## I. Getting started

git clone https://github.com/CarolineHVermette/NEXA-Web.git
cd NEXA-Web
python -m http.server 8000

Then open http://localhost:8000/index.html. An HTTP server is required: the 3D models will not load from a file:// address, because of browser security rules.

Read in this order:

- README.md, the project overview. It was synchronised with V1 on 2026-09-09, so the two agree; where they ever disagree, this guide is current.

- js/config.js. It is the assumptions register in executable form, and reading its comments is the fastest way to understand what the tool claims and why.

- documentation.html in a browser, the public methodology the users read.

- docs_methodology/README.md, the index of the simulation methodology.

- docs_implementation/documentation-revisions/, the working record of the current revision round: decisions, defects, results and emails.

The complete folder and file map is Part III, section 4.

### Editable and generated files

| Touch | Do not touch without reading Part III first |
| --- | --- |
| js/config.js, for any shared constant or label | js/data.js, which is generated, not written |
| The HTML pages, for layout and copy | previous/, which is archived and served by nothing |
| css/styles.css | Templates/*.csv, which are the import layer, not a place to correct a number |

## J. Origin of a displayed number

Everything the tool shows resolves through one of five structures in js/data.js, or through js/config.js.

| Structure | Holds | Feeds |
| --- | --- | --- |
| NEIGHBOURHOODS | The 35 archetypes: code, land use, context, density, layout, composition, building count, and which envelope keys they support | Layer 1 |
| ENVELOPE_ENERGY_DATA | The core. [envelopeKey][NU][scenario] giving total, a six item breakdown and pv. 17 envelope keys, 35 neighbourhoods, 5 scenarios, 2,974 cells | Layer 2, and the final summary |
| GFA_DATA and CONDITIONED_AREA_DATA | Total built floor area, and heated and cooled floor area, per neighbourhood | Absolute totals. Which one, matters |
| EV_V2G_DATA | Per neighbourhood, per scenario, daily charging and vehicle to grid energy | Layer 3 |
| LPV_DATA | The landscape solar configuration and result table | Layer 4 |

js/config.js holds everything else: the climate list, the labels, the solar constants, the scenario names, the status vocabulary, the withheld neighbourhood and climate pairs, and the landscape solar calculation chain.

The floor area rule is the easiest thing in this project to get wrong. Intensities are per heated and cooled area; absolute totals multiply by CONDITIONED_AREA_DATA, never by GFA_DATA. Two separate defects in this round were the same mistake, and both displayed a total that was twice the correct one. Stated in full, with the reason, in Part III, section 5.1. Read it before you write any total.

Every displayed field traced to its source is Part III, section 5, the Results Data Map.

Procedures for the three changes a maintainer makes most often are in Appendix E.

## K. Codebase constraints

| Constraint | Consequence |
| --- | --- |
| js/data.js is about 1.7 MB and loads on every page | First paint carries a real download. Fine on a desktop connection, noticeable on mobile |
| No module system, scripts are plain globals loaded in order | config.js must load before data.js on every page. The order is part of the contract |
| State is kept in sessionStorage | Closing the tab loses the design. There is no save or share of a configuration |
| Every asset is stamped ?v=N | Browsers cache aggressively. If you change a script or the data and do not bump N, the deploy will appear to do nothing |
| No silent climate fallback remains | Removed 2026-09-09 under A15, in the last 16 sites across 8 files. With no climate chosen, a built link now carries an empty envelope and the page it opens states the refusal instead of drawing Montreal |

Case sensitivity is the recurring trap. Development is on Windows, which ignores case; GitHub Pages is Linux, which does not. A path built by interpolating a display label into a filename works locally and returns a 404 live. That is the cause of the broken icons that were reported on the live site and were not reproducible on the developer machine. Every icon path now lives as an explicit string in js/config.js beside its label, and is asserted against the real directory listing.

### Verification

There is no test framework and no continuous integration. Verification is by standalone Node scripts under docs_implementation/documentation-revisions/Results/, one per stage group, which load the real js/data.js and js/config.js, run the real page logic against a stub DOM, and assert against the data rather than against a snapshot. Eight suites, 706 checks, 685 passing as measured on 2026-09-09. All 21 failures are checks written against interface details that changed afterwards; none is a data check, and no data check fails. The two that were a real defect, the mixed line endings of js/pv.js, closed the same day with DBG-051, which is why the figure reads 685 and not the 683 measured earlier in the day. The earlier figure of 696 was measured before the line ending assertions were rewritten. Why, and what to do about it, is Part III, section 10.2.

What the suites cannot reach, and what does. No stub DOM can test a click, the back button, a reload, the print output or whether a 3D model actually draws. Those are tested by driving a real browser against the served site. That pass is what found eight of the nine defects of 2026-08-24, and what it did and found is Appendix F.

# Part III. Reference

This is the part that lets RHLab run the tool without its author. Parts I and II summarise; this one states. All fifteen sections and the three appendices are carried over whole from version 0.8.

## 1. Purpose and scope

This guide is the single reference that allows CHV and RHLab to understand, maintain, update, deploy and continue developing the NEXA-web tool independently, without depending on any personal file, personal account, personal machine or personal memory.

It is deliberately separate from the user facing methodology published at documentation.html. That document explains the science to a stakeholder. This one explains the system to a maintainer. The separation was an explicit instruction, given twice: internal material must not leak into the public methodology, and this guide must not be reduced to a pointer at it.

Test of completeness. The guide is written to make it possible for a new RHLab member, given this guide and repository access and nothing else, to: find where any displayed number comes from, update an existing neighbourhood result, add a new neighbourhood or climate zone, deploy an updated live version, and restore the site if it breaks. The independent handover test in Appendix G is the practical test of this capability; it has not yet been conducted.

What this version does not yet do. Backup and recovery, section 13, has been tested once and in one direction only, by the mirror push of 2026-08-24. Restoring from the mirror has never been rehearsed. Adding a new neighbourhood, section 8.2, is written from the code rather than from a rehearsal. Both are marked in place.

## 2. System description

NEXA, Neighbourhood Energy eXploration & Analysis, is an early stage neighbourhood energy planning and pre feasibility tool. A user selects a neighbourhood archetype and a climate and code context, then applies efficiency measures, on site generation, electric mobility and green infrastructure options, and sees the resulting energy performance against a baseline.

It is a static website: HTML, CSS and plain JavaScript, with all data pre computed and shipped in js/data.js. There is no backend, no database and no build step. Simulation happens offline; the website only reads results.

What it is not. It is not an optimisation engine, not a design tool, and not a substitute for detailed simulation. No formal optimisation method is implemented and none is planned in the current round. The landing page states this in a box, on CHV’s instruction of 2026-08-13.

### Scale

| Quantity | Value |
| --- | --- |
| Neighbourhood archetypes | 35 |
| Envelope and climate keys | 17 |
| Efficiency scenarios per case | 5, plus an ideal thermal load option |
| Stored scenario results | 2,974 |
| Interactive pages | 14 |
| JavaScript modules | 13 |
| External libraries | 1 |

## 3. Repository and deployment

| Item | Value |
| --- | --- |
| Repository | https://github.com/CarolineHVermette/NEXA-Web |
| Owner | CHV account. This is the destination repository; see section 14 for what remains of the transfer |
| Predecessor repository | github.com/orcunkoraliseri/NEXA-web, the original personal repository. Still a configured remote in the working copy |
| Branch used for the live tool | main |
| Hosting | GitHub Pages, served from the repository root of main (served from CarolineHVermette/NEXA-Web since 2026-09-22, at https://carolinehvermette.github.io/NEXA-Web/; section 14.4, T7) |
| .nojekyll | Present at the repository root, and required. Without it, GitHub Pages applies Jekyll processing and silently drops every folder whose name begins with an underscore |
| Custom domain | None. The site is served on the default github.io address |
| Build step | None. What is committed is what is served |
| Deployment latency | One to two minutes after a push, occasionally longer |
| Cache behaviour | Aggressive. Scripts and stylesheets are versioned with a ?v=N query parameter. ?v=16 on 65 tags across 15 pages on the published site, 2026-08-24. Two verification suites assert that every page carries the same stamp and that no other stamp survives, so a page left behind fails a check rather than going out quietly. See section 11 |
| Branch strategy | Currently single branch. For the revision round the recommendation is a revision branch merged to main at each publish, so that main is always the live state |

## 4. Folder and file map

### 4.1 Pages, repository root

| File | Role |
| --- | --- |
| index.html | Layer 0, the entry page. Canonical. The near duplicate layer0_initial.html was archived to previous/ on 2026-08-10 |
| layer1_NUs_selection.html | Layer 1, land use, context, density, layout, climate and envelope filter |
| layer1_output.html | Layer 1, the matching archetype cards with composition, floor area and 3D links |
| layer2_energy_selection.html | Layer 2, efficiency scenario, energy system and generation technology selection |
| layer2_energy_breakdown.html | Layer 2, the demand breakdown treemap |
| layer2_pv_breakdown.html | Layer 2, the solar generation profile |
| layer3_mobility_selection.html | Layer 3, mobility and transport selection |
| layer3_ev_v2g_mobility_output.html | Layer 3, the mobility and vehicle to grid result |
| layer4_green_selection.html | Layer 4, green infrastructure and landscape solar selection |
| layer4_output_selection.html | Layer 4, the green performance result |
| layer4_lpv_breakdown.html | Layer 4, the landscape solar analysis |
| layer4_finish_design.html | The final consolidated summary dashboard |
| 3dviewer.html | Standalone 3D model viewer. The only page with an external dependency |
| documentation.html | The public methodology and references |

There is no Layer 2 output page. layer2_output_energy.html was archived on 2026-08-10: it was titled “Energy Selection Results”, held no energy value, was reachable only by pressing Back, and rendered broken because a div was never closed. Layer 2 now goes straight from the selection screen to the two breakdown pages.

### 4.2 Scripts, js/

| File | Size | Role |
| --- | --- | --- |
| config.js | 120 kB | Single source of truth for every shared constant. Created 2026-08-10, and the six hand-maintained data blocks joined it on 2026-09-09. Must load before data.js on every page |
| data.js | 1.7 MB | All generated tool data. NEIGHBOURHOODS, BUILDING_IMAGES, PV_GENERATION_DATA, ENVELOPE_ENERGY_DATA, ENERGY_DATA_ARCHIVED, LPV_DATA, EV_V2G_DATA. Generated, not written by hand. A result is never typed into js/data.js or into a page; it is produced by the pipeline, or it does not change. On 2026-09-09 the six blocks that were maintained by hand moved to js/config.js: CONCEPTS, ENERGY_COLORS, ENERGY_STATUS_IMAGES, GFA_DATA, CONDITIONED_AREA_DATA and PV_AREA_DATA. That is A12, and it is why this file can now be regenerated without losing anything |
| app.js | 38 kB | Layer 1 filtering, archetype table rendering, navigation |
| finish-design.js | 40 kB | The final consolidated summary |
| energy.js | 31 kB | The demand treemap |
| pv.js | 23 kB | The solar profile page |
| sidebar.js | 19 kB | The persistent selection sidebar, on every page |
| energy-selection.js | 14 kB | Layer 2 selection state |
| lpv.js | 11 kB | Landscape solar analysis |
| ev-v2g-breakdown.js | 10 kB | Mobility and vehicle to grid result, scenario resolution |
| mobility-selection.js | 7 kB | Layer 3 selection state |
| green-selection.js | 4 kB | Layer 4 selection state |
| output_green.js | 2 kB | Layer 4 green output |

Removed from the live set during this round, and why:

- heatmap-data.js, deleted 2026-08-10. It held hourly irradiation data for 5 of the 35 neighbourhoods, layer2_pv_breakdown.html had no canvas to draw into, and the drawing function was never called. It was downloaded on every visit and rendered nothing. The source CSVs under Content/Images_PVpage/RC/ are untouched, so it is recoverable if the other 30 neighbourhoods are ever produced upstream.

- ev.js and output_energy.js, moved to previous/ with the pages they served.

### 4.3 Styles, css/

styles.css is live and is the whole design system. styles.backup.css, styles.v1.backup.css and styles.v2.backup.css are historical and are removal candidates.

### 4.4 Content, Content/

Twenty five image folders plus Glb_Models/ for the 3D models, IFC_Models/ for the BIM interchange models, and References & Methodology/ for the reference PDFs.

Folder and file names contain spaces and mixed case. This is the root cause of the live site icon failures described in section 12. It has been contained by storing every icon path explicitly rather than building it, but the underlying naming is unchanged.

### 4.5 Data pipeline, Templates/

| Path | Role |
| --- | --- |
| scripts/convert_master_csv.py | Campaign master CSV to per climate CSV |
| scripts/patch_data_js.py | Per climate CSV into js/data.js |
| scripts/check_keys.py | Key coverage validation |
| scripts/test_data_flow.py | End to end data flow test |
| scripts/out_data.js | Generated intermediate output |
| CAN_Z4.csv … CAN_Z8.csv, ASHRAE.csv | Per climate energy results, the import layer |
| PV_generation.csv | Solar generation lookup |
| NUS_EV.csv, NUs_LPV.csv | Mobility and landscape solar parameters |
| Interface_Connections.csv | Navigation flow matrix |
| Welcome_Page_Parameters.csv | Landing page parameters |
| 2026-09-10/ | Current dataset, named by the DATA_VERSION constant in convert_master_csv.py: the post-dfix national master, the IAL combined CSV, PV and archetype CSVs |
| 1983-Quebec/, 2026-07-19/, 2026-07-21/ | Earlier dated source datasets, kept as history. 2026-07-19/ is still read, restricted to its US_ASHRAE/ASHRAE rows, since no later dataset carries that standard |
| archive/, *.bak | Historical |

This pipeline is the single most important thing in this guide. It is how new simulation results become tool results, and it is the part most dependent on one person’s knowledge. Section 7 documents it.

All pipeline inputs resolve through one constant. DATA_VERSION in convert_master_csv.py names the current dated folder (2026-09-10). To move to a new campaign: create a new Templates/<YYYY-MM-DD>/ folder, never over an existing one; copy in the national master, PV generation, archetype and (if produced) IAL CSVs, carrying forward any file the new campaign did not re-run; then change DATA_VERSION to that folder’s name. Older dated folders are left in place as history and are not read again, except 2026-07-19/ for US_ASHRAE/ASHRAE.

### 4.6 Documentation folders

| Folder | Content |
| --- | --- |
| docs_methodology/ | New, 2026-08-14. The curated simulation methodology, copied into the repository so citations resolve. See section 6 |
| docs_implementation/documentation-revisions/ | The revision round working record: Implementation/ stage plans, Progress/ decisions and status, Debugs/ the defect register, Results/ verification scripts and evidence, Emails/ the correspondence, Submission/ this guide and, in previous/, the drafts and the two shorter reports it replaces |
| docs/ | The original page by page implementation specifications |
| previous/ | Archived pages, scripts and design images |

## 5. Results Data Map

The required table. Every displayed result, traced to its source.

CHV specified the minimum columns. They are kept, in her order.

| Displayed result | Unit | Web data file | Variable or key | Original source or model | How generated or updated | Used on which page |
| --- | --- | --- | --- | --- | --- | --- |
| Annual energy use intensity | kWh/m2 of heated and cooled area per year | js/data.js | ENVELOPE_ENERGY_DATA[env][NU][scenario].total | EnergyPlus neighbourhood simulation, campaign option_9_j_20260707_v2; Montreal from option_9_qc1983nu_20260807_all35 | convert_master_csv.py then patch_data_js.py | Layer 2 breakdown, final summary |
| Demand breakdown, six end uses | kWh/m2 of heated and cooled area per year | js/data.js | …[scenario].breakdown[], in the order Heating, Cooling, DHW, Lighting, Equipment, Fans and Pumps | EnergyPlus end use output | same | Layer 2 breakdown |
| Ideal thermal load (Thermal Load card) | kWh/m2 of heated and cooled area per year | js/data.js | ENVELOPE_ENERGY_DATA[env][NU]["IAL"].total and .breakdown[], gated by LMN_CONFIG.thermalLoadAllowed(env) | EnergyPlus neighbourhood simulation, HVAC replaced by ideal air loads; campaigns option_9_a1_IAL_20260910_v1 to _v6, one per NECB zone, combined in IAL_ThermalLoadSims_Combined_6Zone_20260910.csv | convert_master_csv.py then patch_data_js.py, imported 2026-09-10, DBG-054 | Layer 2 breakdown, Thermal Load card. Standard NECB envelopes only, 210 of 210 cells; not currently available for the High-Performance envelope |
| “Other” end use block | kWh/m2 of heated and cooled area per year | computed at render | total minus the sum of breakdown | The seven EnergyPlus end use buckets the tool does not chart: exterior lighting, refrigeration, heat rejection, humidification, heat recovery | Computed by js/energy.js, never stored | Layer 2 breakdown |
| Rooftop solar generation intensity | kWh/m2 of heated and cooled area per year | js/data.js | ENVELOPE_ENERGY_DATA[env][NU][scenario].pv | Tier 3 geometry sized solar injector, from EEM1 on. Native prototype solar only at DEFAULT | same | Solar profile, final summary |
| Authoritative solar source |  |  | LMN_CONFIG.pv.authoritativeSource names the field above. PV_GENERATION_DATA[NU].generation was a second, non reproducing figure and was deleted on 2026-08-10 | Verified against the simulation output for all 35: the energy data reproduces it 35 of 35, the deleted field 2 of 35 |  |  |
| Total rooftop solar array area | m2 | js/data.js | PV_AREA_DATA[NU]. 26 of 35 neighbourhoods only; the other 9 keys are absent and js/pv.js hides the row rather than estimating | Not an EnergyPlus output and not in any results CSV. A parse of the Generator:Photovoltaic surfaces in the injected IDF of the EEM4 run, campaign option_9_qc1983nu_20260807_all35, summed per neighbourhood and multiplied by the injector’s active fraction for that roof group, 0.85 pitched and 1.0 flat | Extracted once by the X23 pass, 2026-08-24, and gated: a neighbourhood is listed only if its own EEM4 generation reproduces the shipped ENVELOPE_ENERGY_DATA["necb-z6"][NU]["EEM4"].pv × CONDITIONED_AREA_DATA[NU] to within 2 per cent. The 9 that fail are all pitched roof archetypes, all short in the same direction, logged as DBG-038 and unresolved. Never fill a gap here with an estimate | Solar profile, final summary |
| Solar array parameters: surface, mounting, tilt, efficiency, ground coverage ratio | text | js/config.js | LMN_CONFIG.pv.roofGroups.pitched and .flat, resolved per neighbourhood by roofGroupFor() | docs_methodology/PV/PV_methodology.md sections 3 and 6 | Manual, one place | Solar profile, final summary |
| Absolute solar generation | MWh per year | computed at render | pv intensity × CONDITIONED_AREA_DATA[NU] | Derived | Computed by js/pv.js and js/finish-design.js | Solar profile, final summary |
| Ratio of Performance | dimensionless | computed at render | pv / total | Published metric of the neighbourhood paper | Computed at render time, never stored, so it cannot go stale | Solar profile |
| Heated and cooled floor area | m2 | js/data.js | CONDITIONED_AREA_DATA[NU] | EnergyPlus Net Conditioned Building Area, from LMN1983_NU_validation_all35.csv column area_cond | Manual import, 2026-08-10 | The denominator of every intensity and the multiplier of every absolute total |
| Total built floor area | m2 | js/data.js | GFA_DATA[NU] | EnergyPlus Total Building Area, same source file, column area_gfa | Manual import, 2026-08-10 | Displayed on the neighbourhood table and the solar page. Feeds no calculation |
| Neighbourhood composition and building count | counts | js/data.js | NEIGHBOURHOODS[].content, .buildingCount | The merged sector IDF listing of the campaign | Manual, cross checked three ways in 2026-08-10 | Layer 1 |
| Supported climates per neighbourhood | list | js/data.js | NEIGHBOURHOODS[].envelope[] | Which arms of the campaign completed | Manual | Layer 1 filter |
| Withheld neighbourhood and climate pairs | text | js/config.js | LMN_CONFIG.dataGaps[] | Defect analysis, DBG-027 and DBG-028 | Manual, one place, gated on four pages | Layer 1, Layer 2, solar profile, final summary |
| Electric vehicle daily charging demand | kWh per day | js/data.js | EV_V2G_DATA[NU][EV1\¦EV2].totalEvEnergyDemand | Fixed calculation: households × vehicles per household × daily demand per vehicle | Manual, from Templates/NUS_EV.csv | Layer 3 result, final summary |
| Storage loss | kWh per day | js/data.js | ….storageLoss | Same calculation chain | same | Layer 3 result |
| Vehicle to grid energy available | kWh per day | js/data.js | ….v2gPowerAvailable. Named a power, holds an energy. The display label comes from LMN_CONFIG.ev.dailyV2gEnergyLabel instead | Same calculation chain | same | Layer 3 result |
| Net grid demand, total | kWh per day | js/data.js | ….netEnergyBalance_kWh | Derived | same | Layer 3 result, final summary |
| Net grid demand intensity | kWh/(m2 per day) | js/data.js | ….netEnergyBalance_kWh_m2 | Derived, against the heated and cooled area | Recomputed 2026-08-12 against CONDITIONED_AREA_DATA | Layer 3 result |
| System status | label | js/data.js | ….systemStatusIndicator | Derived. Displayed through LMN_CONFIG.ev.statusStates, never raw | same | Layer 3 result, status image |
| Landscape solar, land and site area | m2 | js/config.js | LMN_CONFIG.lpv.siteAreaM2 = 20,234 | Project decision: a uniform 5 acres for every neighbourhood | Manual, one place | Layer 4 landscape solar |
| Landscape solar, land allocation | % and m2 | js/config.js | landAllocationFraction = 0.20 | Assumption | Manual | same |
| Landscape solar, usable area | % and m2 | js/config.js | usableFraction = 0.10, applied in series with the allocation | Assumption | Manual | same |
| Landscape solar, installed capacity | kWp | js/config.js chain | site × 0.20 × 0.10 × 0.200 kW/m2 = 80.9 kWp | Derived | LMN_CONFIG.lpvChain(), and js/data.js values are generated from it | same |
| Landscape solar, annual generation | MWh per year | js/config.js chain | 80.9 kWp × 1,280 kWh/kWp per year = 103.6 MWh per year | Derived | same | same, and final summary |
| 3D model | GLB file | Content/Glb_Models/ | Filename by neighbourhood code | Morphology model export | Manual | 3D viewer |
| Neighbourhood, building and status images | PNG | Content/Images_*/ | Explicit path strings in LMN_CONFIG.selectionLabels | Project assets | Manual, asserted against the real directory listing with exact case | Every layer |
| Scenario and measure labels | text | js/config.js | LMN_CONFIG.eemLabels, eemDetails, selectionLabels, envelopeLabels | The upstream ladder documentation | Manual, one place | Every layer |

Acceptance criterion. No cell in this table is empty. Three cells that were marked “to confirm” in the v0.1 draft are now filled: the electric vehicle units are daily energy throughout, not power; the vehicle to grid field name is wrong and the display label corrects it; and the landscape solar chain is generated rather than stored.

### 5.1 The floor area rule

There are two floor areas per neighbourhood and they are not interchangeable. On the house neighbourhoods they differ by exactly a factor of two, because the attic and the unheated basement are half the built area and neither is conditioned.

| Quantity | Denominator or multiplier |
| --- | --- |
| Any intensity, kWh/m2 | Heated and cooled area, CONDITIONED_AREA_DATA |
| Any absolute total, kWh or MWh | Heated and cooled area, CONDITIONED_AREA_DATA |
| The area shown on the neighbourhood table | Total built area, GFA_DATA, display only |

Two separate defects in this round were caused by multiplying an intensity by GFA_DATA. Both printed a total that was twice the correct one, on the solar page and again on the final summary. If you write a new absolute total, multiply by the conditioned area.

Floor area ratio is not implemented in V1. FAR is not computed, not stored and not displayed anywhere in the tool. What V1 offers in its place is the pair above, the two floor areas, together with the density and building count carried per neighbourhood in NEIGHBOURHOODS[] and shown in the sidebar. Do not record a FAR request as delivered: density plus floor area is a different quantity, and a reader who needs FAR would have to divide the built area by a site area the tool does not hold.

## 6. Simulation and model sources

### 6.0 Origin of every published number

The NEXA-web tool generates none of its own numbers. Every value on the website was produced offline in a separate project, idf_reader, and transcribed into js/data.js. Nobody can maintain this tool without being able to reach that project, read its methodology, and re run it.

Produced offline, but not all by the same means. Building energy and rooftop solar are EnergyPlus campaign outputs. The two headline indicators are derived from those outputs by calculation, from stated constants. Mobility and vehicle to grid, and landscape solar, are not simulations at all: they are separate fixed calculation chains, marked Preliminary everywhere they appear, and sections 6.8 and 6.9 set out exactly what each one does. Any sentence anywhere that says every displayed number came out of EnergyPlus is wrong, and this section is the one that governs.

Where that project is. It is a second repository, idf_reader, and every part of it a maintainer needs is version controlled and pushed. Earlier versions of this guide said the simulation code, the input models and the raw outputs were “outside, on a personal machine”. That was wrong, and it understated the handover badly. They are in a repository, and that repository already carries a second remote pointing at CHV’s own GitHub account.

|  | Where it is | Contents |
| --- | --- | --- |
| Methodology | ✅ In the website repository, docs_methodology/ | Building sources and modelling assumptions, the Canadian code transformation, the efficiency measure catalogue, the solar model, the campaign specification, the validation tables |
| Simulation code | ✅ In the idf_reader repository, branch main | The EnergyPlus toolkit: the appliers, the injector, the neighbourhood registry, the run harness. The Python modules at the repository root |
| Input models | ✅ Same repository | 4,655 IDF files tracked, including the prototype models and the merged neighbourhood models, under Content/. 33 weather files under Content/WeatherFiles/, the Canadian CWEC2020v2 files among them |
| Raw outputs | ✅ Same repository | 12,734 tracked files under outputs/, and 3,451 CSV files in total, the per building results the master CSVs were built from |

The simulation repository, in full.

| Item | Value |
| --- | --- |
| Repository | github.com/orcunkoraliseri/NEXA-desktop, named idf_reader until 2026-09-28. Private: access by collaborator invitation |
| Branch | main |
| Tracked files | 19,701 |
| Second remote | github.com/CarolineHVermette/LMN-Desktop, on CHV’s account |
| State on 2026-08-24 | Local main, origin/main and the remote on CHV’s account all agree. It was two commits behind that morning, 5a1ee23b against ee32e462, and was pushed the same afternoon |

What is not in that repository, and why it still matters. The working directory on the machine is roughly 500 GB, of which the repository is a small part. The bulk is untracked: 0_BEM_Setup at about 257 GB and tmp at about 197 GB. Most of it is intermediate run space, but not all of it. 0_BEM_Setup/SimResults_neighbourhoods/, about 252 GB, holds the raw neighbourhood campaign runs, and those exist in one copy on one machine. They are not needed to re run the campaign, and they are the only record of the runs that produced the published numbers. Section 13.2 sets out the four classes and what would be lost; archiving that folder is item A5 of the 2026-09-09 closeout checklist and is not done. Nothing of the true scratch belongs in a repository, and none of it belongs in the website repository, which serves GitHub Pages and is capped at 1 GB.

So the transfer that was called T12 was smaller than this guide claimed, and it is done. It was never a copy of half a terabyte. It was one push to bring CHV’s remote up to date, and that push went out on 2026-08-24. What remains is her confirmation that she can read both repositories.

### 6.1 EnergyPlus models

| Item | Value |
| --- | --- |
| Engine | EnergyPlus 22.1.0 for the campaign that produced this website’s data |
| Version pinning | Each source model is pinned to the version it was authored for, detected from its own Version field. Pinning one engine across a mixed set fails within a second with fatal errors |
| Campaign, seven climate arms (five published, Zone 8 and ASHRAE) | option_9_j_20260707_v2 |
| Campaign, corrected Montreal | option_9_qc1983nu_20260807_all35 |
| Both recorded in | LMN_CONFIG.dataCampaign |

### 6.2 Building prototypes

| Source | Count | Used for |
| --- | --- | --- |
| ASHRAE 90.1-2022 DOE and PNNL commercial prototypes, Buffalo NY base form | 16 | Commercial, mixed use, institutional |
| IECC 2024 single family, Climate Zone 6A, three variants | 3 | Housing |
| ASHRAE 90.1-2019 data centre prototypes | 2 | Data centre neighbourhoods |
| OpenStudio and DOE supplementary buildings | 7 | College, Supermarket, Laboratory, Tall, SuperTall, two small data centres |
| Project developed near zero energy retail | 1 | A high performance archetype not covered by the standard sets |

Three prototypes are used at 50 per cent floor area to fit neighbourhood plots: Primary School, Secondary School and Warehouse. Full detail: docs_methodology/BEM/BEM_methodology.md section 3, and docs_methodology/BEM/Resizing_methodology.md.

### 6.3 Weather files

Every published climate arm is driven by a Canadian weather file. The five published arms each use the CWEC2020v2 file of their representative city, named on the result page and in the assumptions block of the tool itself. The 33 weather files are tracked in the simulation repository under Content/WeatherFiles/.

| Published climate arm | Weather file |
| --- | --- |
| Vancouver, NECB Zone 4 | CAN_BC_Vancouver.Intl.AP.718920_CWEC2020v2.epw |
| Toronto, NECB Zone 5 | CAN_ON_Bishop-Toronto.City.AP.712650_CWEC2020v2.epw |
| Montreal, NECB Zone 6 | CAN_QC_Montreal-Trudeau.Intl.AP.716270_CWEC2020v2.epw |
| Winnipeg, NECB Zone 7A | CAN_MB_Winnipeg-Richardson.Intl.AP.718520_CWEC2020v2.epw |
| Fort McMurray, NECB Zone 7B | CAN_AB_Fort.McMurray.AP.716890_CWEC2020v2.epw |

A note on the prototype file names. The DOE and PNNL prototype library contains no Canadian city, so the American prototype names survive inside the file names of the Canadian runs. The name records the geometry the model started from, before the Canadian code transformation of section 6.4 was applied. The weather file that drove the simulation is the Canadian one listed above. Full rationale, for a maintainer who needs it: docs_methodology/BEM/ClimateZone6ASelection_Reason.md.

### 6.4 The Canadian transformation

Every US prototype is converted to the Canadian codes before simulation.

|  | Commercial, institutional, industrial | Housing |
| --- | --- | --- |
| Code | NECB 2017 | NBC 9.36 |
| Wall U cap, Zone 6 | 0.247 W/m2K | NBC 9.36 table, wood frame |
| Roof U cap, Zone 6 | 0.183 W/m2K | NBC 9.36 table |
| Window U cap, Zone 6 | 1.90 W/m2K | 1.60 W/m2K |
| Air tightness | 0.25 L/s per m2 at 75 Pa | 2.5 air changes per hour at 50 Pa |
| Lighting power density | Clamped by space type, NECB Table 4.2.1.6 | Not clamped, governed by EnerGuide labelling |
| HVAC efficiency floors | NECB section 5.2 minimums | Not clamped, same reason |
| Internal loads and schedules | BTAP Canadian occupancy, plug load and thermostat schedules | Prototype native |

Every clamp uses skip when better: a component that already meets the Canadian cap is left untouched. A US prototype that was already better than NECB therefore shows no change from the transformation. That is correct behaviour, not a bug. Full detail: docs_methodology/BEM/CAN_transformation.md.

### 6.5 Energy Efficiency Measures

The website’s five rungs are the campaign’s scenario tags. The mapping is recorded in LMN_CONFIG.eemCampaignTags so a rung can always be traced back. The Baseline rung is a code-compliant reference case, a new neighbourhood built to NECB 2017. It is not a survey of existing buildings and it is not the current condition of any real neighbourhood.

| Website rung | Campaign tag | Contains |
| --- | --- | --- |
| Baseline (reference case) | EEM_J_DEFAULT | Code-compliant reference case as designed. Native solar only |
| + Envelope | EEM_J_ENVELOPE | High performance opaque assemblies, triple glazing U 0.85 SHGC 0.40, foundation and slab insulation in the cold zones, infiltration reduced to a quarter |
| + Envelope, heat pump | EEM_J_ENV_HVAC | Adds a cold climate air source heat pump, cold climate performance curves, inverter cooling, per zone units in housing, on demand defrost, backup lockout |
| + Envelope, heat pump, hot water | EEM_J_ENV_HVAC_DHW | Adds a transcritical CO2 heat pump water heater, stratified tank, outdoor air intake, off peak boost, R-25 jacket |
| + Envelope, heat pump, hot water, lighting, equipment and cooling | EEM_J_ENV_HVAC_DHW_EEM4 | Adds automated blinds, daylight dimming, LED lighting, occupancy trim, ENERGY STAR plug loads, gas appliance electrification in housing |

Each rung is a strict superset of the one below, which is why each names its whole package rather than only what it adds. The provisional label that called the last rung a “deep retrofit” was wrong and was corrected: it adds lighting, equipment and cooling, not a deeper envelope. Measure by measure catalogue, with the published reference behind each measure and the buildings it is skipped on: docs_methodology/EEM/EEM_setup.md.

### 6.6 The solar model

Two tracks:

| Track | Behaviour |
| --- | --- |
| Baseline | Whatever solar the source prototype ships with. Zero for every Canadian baseline, since neither NECB 2017 nor NBC 9.36 ships native solar |
| Every other rung | A geometry aware injector sizes an array from the actual roof |

|  | Pitched roofs, four house neighbourhoods | Flat roofs, the other thirty one |
| --- | --- | --- |
| Neighbourhoods | RC-R, RC-D, RC-ML, RC-T | All others |
| Mounting | Flush on the south facing roof face | Fixed open rack |
| Tilt | Follows the roof pitch | 45 degrees, facing south |
| Panel efficiency | 23 per cent | 23 per cent |
| Active fraction of the reference surface | 0.85 of the south roof face | 1.0 on the rack |
| Ground coverage ratio | Does not apply | 0.40 |
| System losses | 14 per cent | 14 per cent |
| Inverter efficiency | 96 per cent | 96 per cent |

Corrected on 2026-08-24. This table previously read 18.65 per cent for flat roofs and 20 per cent of aperture for pitched. Neither produced a number in this tool. Every published PV value came from the Tier 3 injector at a panel efficiency of 0.230, fixed in BEM_utils/pv_tier3.py line 50 and selected in main_BEM.py line 757. The 18.65 per cent is a cell efficiency from the Tier 1 injector, retired on 2026-05-29, sitting behind an active fraction of 0.80. The 20 per cent is a module efficiency of aperture area behind a 0.90 fraction, quoted in the PV methodology as a literature reference. Comparing the two without their active fractions compares a cell with a module, which is why the difference never closed. On the surfaces actually used: pitched, 0.230 x 0.85 gives 19.55 per cent of the south roof face; flat, the cells fill a 45 degree rack whose area is the roof area times a ground coverage ratio of 0.40 divided by cos 45. No simulation was re-run and no stored value moved. Only the assumption printed beside the results was wrong.

Group membership is not guessed: the four are exactly the neighbourhoods whose solar ratio falls in the 1.68 to 1.73 band that the 2026-05-29 south face only fix explains. RS-S has an 18.8 degree mean roof tilt but sits in the flat roof band and the pipeline treats it as flat.

The rooftop figure is identical on every rung, by design. The injector sizes the array from the roof, and no envelope or heat pump measure changes the roof. The page says so, because without that sentence a user clicking through the ladder sees a frozen number and reasonably concludes the page is broken.

Snow cover is not modelled. Rooftop solar is optimistic by roughly 10 to 15 per cent in Zone 7A, 15 to 20 per cent in Zone 7B and 20 to 25 per cent in Zone 8. The numbers are not changed; a note is shown in those three zones. Full model: docs_methodology/PV/PV_methodology.md.

### 6.7 Facade solar

Tested upstream on three single buildings, Montreal only. No neighbourhood level facade result exists. It is therefore restricted to the nine Montreal neighbourhoods containing one of those three archetypes, labelled preliminary, and excluded from the totals. Outside those nine the card is greyed with a badge reading “Montreal only” and a line saying why.

### 6.8 Mobility and vehicle to grid

Not a simulation. A fixed calculation chain: households x vehicles per household x (daily charging demand / charging efficiency), where charging efficiency grosses the demand up (demand = households x 1.5 x (15 / 0.90)), it does not reduce it. The whole of Layer 3 is marked Preliminary in the interface and in this guide.

On disk, the six calculation values are the default arguments of Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py. The equations in that file are numbered 12 to 18, so they were transcribed from a document whose equations 1 to 11 came first. That document is identified, 2026-08-31:

Hachem-Vermette, C. (2025). Designing energy-positive neighborhoods: a modular framework for integrated planning and policy guidance. Energy Reports, 14, 4492 to 4507. Section 2.4, equations 12 to 18. Local copy: docs_implementation/documentation-revisions/Resources/1-s2.0-S2352484725006365-main.pdf.

The match is exact, symbol for symbol, for all seven equations, and equations 1 to 11 of the paper are the photovoltaic, district heating and heat pump chain, which is why the electric vehicle block starts at 12. The earlier bibliographic search, sixteen publications with four read in full text, did not reach it and is kept for the record in docs_implementation/documentation-revisions/DeepResearchPrompts/responses/RT02_chv_ev_v2g_source_paper.md. The 15 kWh per EV daily demand carries a stated basis in Templates/NUS_EV.csv, a 200 km daily range, and section 2.4 of the paper states the same figure and cites Dalla Chiara et al. (2019). Dr. Hachem-Vermette’s instruction of 2026-08-30 was to use her paper’s assumptions as the provisional basis, and the paper is now named. As noted in her instruction of 2026-08-30, the V2G methodology, efficiencies and losses will require further development and validation in future work.

Both method flags are now closed, by CHV’s decisions of 2026-09-09 (her closeout checklist, items E1 to E3). Her ruling on the module as a whole comes first: the paper-based chain is retained as a provisional basis for V1, the whole of Layer 3 stays marked Preliminary, and the methodology, the efficiencies and the loss assumptions are named as requiring future development and validation. Neither flag is redesigned now. Both become stated V1 limitations:

- f1, the discharge efficiency: closed 2026-09-09. Her decision: “Do not claim that the 90 per cent numeric value comes from the paper. It is a provisional code assumption. The current export equation does not apply a discharge-efficiency term; state this transparently as a V1 limitation and future validation item rather than redesigning the module now.” The 90 per cent therefore stays where it is, is not applied, and is a limitation of V1, not an open question.

- f2, the stationary storage share: closed 2026-09-09. Her decision: retain the 50 per cent provisionally because it is the paper’s own value, state that it is an assumption and not a validated universal value, and treat the resulting storage loss figure as Preliminary and a subject for future work.

The two flags as they were recorded, and the evidence behind them, are kept below because the reasoning is what a future maintainer needs:

- The discharge efficiency is displayed but not applied to the exported energy. The page states this rather than implying a hundred per cent discharge. The paper does the same: its equation 17 subtracts the vehicle to grid discharge without an efficiency term, and it never gives the battery efficiency a numeric value. The 90 per cent in the tool is a default argument, not a figure read from the paper. Whether to apply it was open flag f1, closed on 2026-09-09: it is not applied, and that is a stated V1 limitation.

- The storage loss of 5 per cent is derived from an assumed 50 per cent stationary storage share (calculate_ev_scenarios.py line 31). That share is the paper’s own, written inside equation 13, and the paper sets it without justification: section 2.4.1 says only that a portion of the demand is routed through stationary storage. No literature convention sets it either. A candidate physical reading, not confirmed and not the origin of the constant, is that the 5 per cent stands in for the standby drain of a parked vehicle, 10 to 40 W continuously, which is 0.25 to 0.80 kWh per vehicle per day against the tool’s 0.83. Whether to relabel the term on that basis was open flag f2, closed on 2026-09-09: the share is retained as the paper’s own value, called an assumption rather than a validated one, and the loss it produces is Preliminary.

Sign convention, which must be stated in words on screen because it collides with a common one: positive means the grid must supply, following the grid delivery convention. In the net zero and positive energy district literature, positive usually means surplus.

### 6.9 Landscape solar

Of the eight Layer 4 green infrastructure options, only one is quantified (1 of 8):

| Group | Options | Quantified? |
| --- | --- | --- |
| infrastructure | Green Roofs, Vertical Greening Systems, Linear Greenery, Green Spaces | No |
| urbanAgriculture | Roof Gardens, Food Gardens | No |
| energyIntegrated | PV-Green Roofs Integrated Modules | No |
| energyIntegrated | Landscape PV | Yes, and it is a fixed arithmetic chain, section 6.9, not EnergyPlus |

Not a simulation. A fixed land area assumption multiplied by a specific yield. The chain, and it is now written in exactly one place:

20,234 m2 site          (a uniform 5 acres, by decision, for every neighbourhood)
  × 0.20 allocation  =   4,046.8 m2 allocated
  × 0.10 usable      =     404.7 m2 usable      (in series, not in parallel)
  × 0.200 kW/m2      =      80.9 kWp installed
  × 1,280 kWh/kWp    =     103.6 MWh per year

The uniform allocation is a project decision, not a defect, and the page says so, because an identical value on 35 neighbourhoods otherwise reads as a bug. The 10 per cent usable fraction is below the published range for fixed tilt ground mount, 30 to 50 per cent; raising it is a redesign of the facility, not a correction, and belongs to a later round.

### 6.10 Modelling choices held out of the public methodology

| Choice | Effect | Why it is internal |
| --- | --- | --- |
| AirflowNetwork without distribution at neighbourhood scale | Duct leakage is dropped. Estimated at most 5 per cent absolute effect on intensity. All neighbourhood variants use the same mode, so relative comparisons stay valid. Wall time drops roughly tenfold | It is a solver stability decision. At neighbourhood scale the duct solver goes singular in high wind weather and the run dies |
| Ground coupling is not harmonised | 23 of the 35 archetypes combine more than one ground boundary method inside a single neighbourhood. The split runs through the density ladder, so comparing a low density archetype against a high density one crosses a method boundary | Inherited from the source prototypes; nobody chose it. Measured worth 1.8 to 2.4 per cent of site energy on the one archetype tested under both models |
| The ground temperature series is lagged Buffalo air temperature | About 12 K too warm through the heating season, so heating is understated for the affected buildings. The annual mean looks correct, so an annual sanity check does not catch it | Not a project error and not a prototype error: the EnergyPlus specification prescribes exactly this. The defect is in the prescription |
| Electrochromic and already performant glazing are preserved | Windows that already beat the high performance target are left alone | Replacing them would raise solar gain and increase cooling by up to 54 per cent on small internally loaded buildings, and would orphan the switchable glazing control |

How to answer a reviewer on the ground coupling today, before anything is re run: state the ground model per building family, state the direction of the error, heating understated on the affected buildings, and give the measured magnitude. Do not claim the models share identical boundary conditions across the archetype series, because they do not. Full account: docs_methodology/BEM/NUs_Setup_Matrix.md sections 3 and 6.

### 6.11 Superseded results

Five compact residential archetypes, RC-D, RC-ML, RC-MR1, RC-R and RC-T, were merged upstream from a US detached house prototype rather than from the Canadian baseline, by a silent fall through in the baseline resolver. The upstream project fixed the code and re simulated. For a period, the corrected results were not imported into js/data.js.

Montreal was corrected on 2026-08-10 from the corrected campaign, 800 of 800 fields verified with zero unexpected changes. The other five NECB climates have no corrected campaign, so those 25 neighbourhood and climate pairs are withheld on CHV’s decision of 2026-08-13 not to launch a re run campaign yet. The wording “results under revision” was removed from the interface on 2026-08-24 at her request; the affected cases are now simply not offered.

Two whole climate arms have since been withdrawn from the public tool, and neither was deleted. The ASHRAE arm was withdrawn on 2026-08-17, on her point that it is a United States reference case on a United States weather file rather than a Canadian climate. NECB Zone 8, Chisasibi, was withdrawn on 2026-08-24, on her instruction that it stay in the research database and the technical documentation without its quantitative results being exposed. Both remain in the CSV set and in this guide. Five arms and 154 neighbourhood and climate pairs are published. Restoring either one is a change in js/config.js, not a re-run.

The lesson for section 8. An update procedure that only answers “how do I change a number” is not enough. It must also answer “how do we know the website is showing the current run”. That is what the campaign stamp in LMN_CONFIG.dataCampaign and the Results Data Map in section 5 are for.

### 6.12 Naming and organisation of the simulation files

Status: In progress (pattern settled on 2026-08-27; rename pass has not been run)

To ensure that a future researcher can identify the NU/building, climate, envelope/scenario, IDF name, weather file and corresponding outputs without opening the file, standard naming patterns were settled on 2026-08-27 (recorded in docs_implementation/documentation-revisions/Submission/previous/PROPOSAL_Simulation-File-Naming_v0.3.md).

#### The two naming patterns

1. Building models:

{library}_{prototype}_{source}_{code}_{zone}_{vintage}_{engine}.idf

- library: Prototype library (ASHRAE901).

- prototype: Building type (ApartmentHighRise, OfficeMedium, Warehouse, etc.).

- source: Source prototype location the geometry and loads came from (Buffalo), kept for provenance.

- code: Code applied (NECB17, QC1983, ASHRAE901).

- zone: Climate zone (Z4, Z5, Z6, Z7A, Z7B, Z8, US), mapping directly to Canadian CWEC weather files.

- vintage: Envelope vintage (STD2022, 1983).

- engine: EnergyPlus engine version (v221).

Example: ASHRAE901_ApartmentHighRise_Buffalo_NECB17_Z6_STD2022_v221.idf

2. Neighbourhood models:

NU_{code}_{climate}_{scenario}_{engine}.idf

- NU: Fixed prefix indicating a neighbourhood model.

- code: Neighbourhood unit archetype code (CC-B, RC-HR2, MU-C1, etc., 35 total).

- climate: Combined code and zone (NECB17-Z6, QC1983-Z6, ASHRAE901-US), mapping to CWEC weather files.

- scenario: Efficiency scenario rung (BASE, HPERF, HPERF-HP, HPERF-HP-DHW, HPERF-HP-DHW-LEC).

- engine: Engine version (v221).

Example: NU_CC-B_NECB17-Z6_BASE_v221.idf

#### Recovery of the six fields

| Required field | How it is recovered from the model name and mapping |
| --- | --- |
| NU / Building | Recovered directly from {code} in neighbourhood models or {prototype} in building models |
| Climate | Recovered directly from {zone} or {climate} |
| Envelope / Scenario | Recovered from {vintage}/{code} in building models and {scenario} in neighbourhood models |
| IDF file name | Stated explicitly as the model file name |
| Weather file | Recovered by mapping the {zone} code to the standard CWEC weather file table below |
| Corresponding outputs | Recovered via the unique timestamped campaign run folder (e.g. {NU}_{EEM}_{TAG}_{TIMESTAMP}) referenced in LMN_national_NU_master.csv |

#### Climate zone to weather file mapping

| Zone code | Representative city | Weather file |
| --- | --- | --- |
| Z4 | Vancouver (NECB Zone 4) | CAN_BC_Vancouver.Intl.AP.718920_CWEC2020v2.epw |
| Z5 | Toronto (NECB Zone 5) | CAN_ON_Bishop-Toronto.City.AP.712650_CWEC2020v2.epw |
| Z6 | Montreal (NECB Zone 6) | CAN_QC_Montreal-Trudeau.Intl.AP.716270_CWEC2020v2.epw |
| Z7A | Winnipeg (NECB Zone 7A) | CAN_MB_Winnipeg-Richardson.Intl.AP.718520_CWEC2020v2.epw |
| Z7B | Fort McMurray (NECB Zone 7B) | CAN_AB_Fort.McMurray.AP.716890_CWEC2020v2.epw |
| Z8 | Chisasibi (NECB Zone 8; withdrawn 2026-08-24 on Dr. Hachem-Vermette’s instruction) | CAN_QC_La.Grande.Riviere.AP.718270_CWEC.epw (Proxy: Chisasibi has no CWEC station; Zone 8 uses the La Grande Riviere / CYGL proxy, station 718270, cited in idf_reader/docs_DONE/docs_LMN_web/LMN-1983/DeepResearch/buildingNECB4-8/00_MASTER_BRIEF.md lines 40, 50. Note that the file vintage is _CWEC, not _CWEC2020v2) |
| US | Buffalo (ASHRAE reference; withdrawn 2026-08-17 as US reference case on US weather file) | USA_NY_Buffalo-Greater.Buffalo.Intl.AP.725280_TMY3.epw |

#### Four-step run order for execution

The rename pass touches 4,655 input models outside this repository and must be executed in the following strict order: 1. Archive raw campaign output: Archive the raw campaign output (0_BEM_Setup/SimResults_neighbourhoods/, 27 campaigns, 3,169 run folders) off the machine first, because it is the only copy. 2. Dry run rename pass: Run a scripted dry run over the 4,655 input models and review the diff before any files are modified. 3. Rewrite CSV provenance paths: In the same pass, rewrite the 2,951 provenance paths in LMN_national_NU_master.csv. Step 2 without step 3 breaks the provenance trail. 4. Re-run verification checks: Re-run existing checks and verify that published website figures remain identical.

Note on current state: The rename pass has not yet been executed. The patterns and procedures are settled for future execution.

### 6.13 The Simulation Model Index

Written 2026-09-09, closeout item A9. Section 6.12 tells you how to read a file name. This is the file that means you do not have to. One row per simulation run behind the published tool, searchable in any spreadsheet:

docs_implementation/documentation-revisions/Submission/submission1/N-LENS_Simulation-Model-Index.csv

The paths in the index point into the simulation repository, idf_reader, not into NEXA-web. idf_path and output_path are paths inside an idf_reader clone: the models and the raw run folders live there, and NEXA-web holds only the published results. output_path is under idf_reader/0_BEM_Setup/SimResults_neighbourhoods/, which is not on GitHub (section 13.2).

4,116 rows, 35 neighbourhoods, 7 climate and envelope arms, built 2026-09-09. Nineteen columns, and every one of them comes from a source rather than from a judgement:

| Column | What it answers | Where it comes from |
| --- | --- | --- |
| nu_code, nu_family | Which neighbourhood | The master CSV |
| component_buildings, component_count, source_prototypes | What it is made of, and which prototype IDF each part came from | neighbourhood_registry.py, upstream |
| code_reference | NECB 2017, ASHRAE 90.1-2022, or the Quebec 1983 reference envelope | The arm |
| climate_key, climate_zone, climate_city, weather_file | Which climate, and the exact weather file | LMN_CONFIG.climates, the tool’s own single source |
| scenario_tag | Which rung of the ladder | The master CSV |
| energyplus_version | 22.1 | Section 9.3 |
| campaign_id | Which campaign folder holds the run | Parsed out of the provenance path |
| idf_path, output_path | The model and its outputs on disk | The master CSV |
| n_severes | Whether EnergyPlus reported severe errors on that run | The master CSV |
| provenance_complete | Whether this row can be traced to a model and an output folder | Computed |
| publication_status | published, withheld with its defect reference, or withdrawn | LMN_CONFIG.dataGaps, withdrawnClimates, and NEIGHBOURHOODS[].envelope |
| source_master_csv | Which master CSV the row was read from | The build |

What the index says about the state of the evidence, and it is worth reading before the rename pass of section 6.12 is run:

|  | Rows |  |
| --- | --- | --- |
| Published | 3,309 | Reachable in the public tool |
| Withheld | 315 | 304 on DBG-028, the five compact residential archetypes; 11 on DBG-027 |
| Withdrawn | 492 | The ASHRAE arm, research only |
| Provenance complete | 2,951 | Both an IDF path and an output path. This is exactly the count of provenance paths section 6.12 says the rename pass has to rewrite, arrived at independently |
| Provenance incomplete | 1,165 | The Quebec 1983 arm records an output path but no IDF path, and 45 corrected Montreal rows record neither |

Rebuild it after any campaign import, in two commands from the repository root:

node docs_implementation/documentation-revisions/Submission/tools/dump_publication_state.js > pubstate.json
python docs_implementation/documentation-revisions/Submission/tools/build_simulation_model_index.py pubstate.json

The first executes js/config.js and js/data.js and writes out what the website itself considers published; the second joins that to the master CSVs and the upstream registry. Standard library only, no EnergyPlus, and it does not touch the 252 GB campaign store.

## 7. Processing scripts

The path from raw simulation output to the data the website reads. A result is never typed into js/data.js or into a page. It is produced by the pipeline, or it does not change (js/data.js is generated, and hand edits are silently discarded on the next run).

EnergyPlus campaign  (outside this repository)
      master_summary.csv per climate arm
            │
            ▼
  Templates/<dated folder>/
            │   Templates/scripts/convert_master_csv.py
            ▼
  Templates/CAN_Z4.csv … CAN_Z8.csv, ASHRAE.csv, PV_generation.csv
            │   Templates/scripts/patch_data_js.py
            ▼
  js/data.js
            │   Templates/scripts/check_keys.py
            │   Templates/scripts/test_data_flow.py
            ▼
  the website

| Script | Input | Output | How to run |
| --- | --- | --- | --- |
| convert_master_csv.py | The Templates/<DATA_VERSION>/ folder (national master, IAL combined CSV, PV and archetype CSVs), plus Templates/2026-07-19/ restricted to US_ASHRAE/ASHRAE rows | A pivoted per climate CSV in Templates/, with an IAL level on the 210 covered cells | python Templates/scripts/convert_master_csv.py |
| patch_data_js.py | The per climate CSVs | Patches the generated structures into js/data.js in place | python Templates/scripts/patch_data_js.py |
| check_keys.py | js/data.js | Reports missing or unexpected keys | python Templates/scripts/check_keys.py |
| test_data_flow.py | js/data.js and the CSVs | End to end consistency check | python Templates/scripts/test_data_flow.py |

All four scripts derive their paths from their own file location (os.path.dirname(os.path.abspath(__file__))), so each is runnable from any working directory on any clone, not only from the repository root.

The scripts use only the Python standard library: csv, json, os, re, sys. There is no requirements file because there are no third party requirements.

### 7.1 Manual steps

This is the highest risk part of the handover. Each of these is done by hand today, and each has caused a defect.

- Bumping DATA_VERSION. convert_master_csv.py resolves every pipeline input through this one constant, currently 2026-09-10. Nothing enforces that it names the current campaign; it must be changed by hand when a newer dated folder is added. This is exactly what produced the superseded results in section 6.11. Record the campaign identifier in LMN_CONFIG.dataCampaign as part of the import, every time.

- Importing the floor areas. GFA_DATA and CONDITIONED_AREA_DATA live in js/config.js since 2026-09-09. They come from a validation CSV, not from the master CSV, and are copied in by hand. They must be taken from the same file, columns area_gfa and area_cond, so they cannot drift apart.

- Importing the composition strings and building counts. Taken from the merged sector listing of the campaign, by hand.

- Updating the neighbourhood and climate support lists. Which arms completed for which neighbourhood is a manual edit to NEIGHBOURHOODS[].envelope[], and a manual entry in LMN_CONFIG.dataGaps for anything withheld.

- Bumping the ?v= cache parameter. Manual, on 65 tags across 15 pages, and if it is forgotten the deploy appears to do nothing. Two verification suites assert that every page carries the same stamp, so a page left behind fails a check rather than going out quietly.

- The mobility and landscape solar tables are not produced by this pipeline at all. They are calculated separately and entered by hand.

## 8. Update procedures

GOVERNING RULE: Results are updated through the pipeline, never on the page. A result is never typed into js/data.js or into a page HTML file. It is produced by the pipeline from simulation outputs, or it does not change. js/data.js is generated: any hand edit is silently discarded the next time the pipeline runs.

The distinction: A result comes from the pipeline; a label describing a result is edited in js/config.js. Both appear on screen, but only the label may be typed by hand.

What you want to change, and where it is done. Each row names the procedure below, and each procedure names the files. They all end the same way: a change reaches the live tool only through section 11, and until that is done it exists on one machine.

| What you want to do | Procedure | What is edited |
| --- | --- | --- |
| Add or update simulation results | 8.1 | Templates/, then pipeline scripts, which write js/data.js, bump ?v= (11.1), publish (11) |
| Add a neighbourhood or a typology | 8.2 | upstream simulation, pipeline, NEIGHBOURHOODS and data tables in js/data.js, Content/, bump ?v= (11.1), publish (11) |
| Add a climate or a location | 8.3 | upstream simulation, pipeline, js/config.js, layer1_NUs_selection.html, bump ?v= (11.1), publish (11) |
| Add an efficiency scenario | 8.4 | upstream simulation, pipeline, LMN_CONFIG.eemLabels, Layer 2 selection page, bump ?v= (11.1), publish (11) |
| Add a technology option | 8.5 | LMN_CONFIG.selectionLabels, bump ?v= (11.1), publish (11) |
| Update an image or a 3D model | 8.6 | Content/, bump ?v= (11.1), publish (11) |
| Modify a simulation assumption (envelope, COP, infiltration, weather, schedules) | 8.7, 8.1 | upstream model in idf_reader, Templates/, pipeline scripts, js/data.js, bump ?v= (11.1), publish (11) |
| Modify a post-processing assumption (the EV chain, the LPV chain, a solar efficiency) | 8.7, 15.0 | the calculation source, re-run it, regenerate its block in js/data.js, bump ?v= (11.1), publish (11) |
| Modify a display assumption (unit labels, basis captions, vocabulary, notes) | 8.7 | js/config.js, bump ?v= (11.1), publish (11) |
| Update text, a table or a figure shown in the interface | 8.8 | js/config.js for a label or quoted number, page HTML for fixed prose, Content/ for assets, bump ?v= (11.1), publish (11) |
| Add a layer or a module, or connect another tool | 8.9 | a new page, a new script in js/, js/sidebar.js, linking page, bump ?v= (11.1), publish (11) |
| Change how the interface looks (colours, fonts, cards/buttons, layout, charts) | 8.10 | css/styles.css tokens, page HTML, js/sidebar.js, bump ?v= (11.1), publish (11) |

### 8.1 Update an existing neighbourhood’s results

Status: To be tested (closes in the Appendix G test, task 6)

The update process follows a strict five-stage chain: 1. Simulation: Run the EnergyPlus simulation campaign upstream (in idf_reader) and obtain the campaign master CSV. 2. Templates/: Create a new Templates/<YYYY-MM-DD>/ folder, copy in the master (and IAL, PV and archetype CSVs where produced), and point DATA_VERSION in convert_master_csv.py at it. 3. Pipeline script: Run convert_master_csv.py to create per-climate CSVs, then run patch_data_js.py. 4. js/data.js: The pipeline updates js/data.js. Verify with check_keys.py, test_data_flow.py, and stage verification scripts. 5. ?v= bump and publish: Bump the cache parameter across pages (section 11.1) and publish to GitHub (section 11).

#### Step by step procedure

- Obtain the campaign output and note its identifier.

- Create Templates/<YYYY-MM-DD>/, never over an existing folder, and copy in the master CSV (and IAL, PV and archetype CSVs where the campaign produced them, carrying forward any file it did not re-run).

- Change DATA_VERSION in convert_master_csv.py to the new folder’s name.

- Run convert_master_csv.py, then patch_data_js.py. Both are runnable from any working directory.

- Update LMN_CONFIG.dataCampaign with the new identifier.

- Run check_keys.py and test_data_flow.py.

- Run the stage verification scripts under docs_implementation/documentation-revisions/Results/ with node.

- Open the affected neighbourhood on the Layer 2 breakdown and on the final summary. Those two pages read the same stored cell by different routes, so disagreement between them is the fastest signal that something is wrong.

- Bump ?v= and publish, section 11.

Never hand edit a value in js/data.js to correct it. The next import overwrites it and nothing records that it was ever changed.

### 8.2 Add a new neighbourhood

Status: To be tested (Appendix G task 5)

- Simulate it upstream across the climate arms you intend to publish.

- Import through 8.1, which will add its rows to ENVELOPE_ENERGY_DATA.

- Add an entry to NEIGHBOURHOODS with: code, conceptId, usage, context, density, layout, the envelope list of supported climate keys, content, buildingCount, image, buildings.

- Add its floor areas to GFA_DATA and CONDITIONED_AREA_DATA in js/config.js, from the same validation file.

- Add its column to LPV_DATA.columns and a value in every row.

- Add its entry to EV_V2G_DATA for both mobility scenarios.

- Add the neighbourhood image under Content/Images_Neighbourhoods/ and the 3D model under Content/Glb_Models/, matching case exactly.

- Decide its solar roof group: if it is a pitched roof archetype, add it to LMN_CONFIG.pv.roofGroups.pitched.nus.

- Run every check in 8.1 step 5 to 7.

### 8.3 Add a new climate zone

Status: To be tested (written from the code, not rehearsed)

- Upstream simulation: Simulate the full 35 neighbourhood set for that zone upstream in EnergyPlus using the appropriate CWEC weather file before modifying configuration.

- Import, which creates a new envelope key in ENVELOPE_ENERGY_DATA.

- Add the zone to LMN_CONFIG.climates with its key, city, zone and standard.

- Add its display name to LMN_CONFIG.envelopeLabels, and its high performance twin.

- Add both keys to LMN_CONFIG.baselineEnvelope and LMN_CONFIG.climateOf.

- Add the zone’s card to layer1_NUs_selection.html.

- Add the climate key to NEIGHBOURHOODS[].envelope[] for every neighbourhood that has a completed run, and add a dataGaps entry for any that does not.

- If snow loss is material in that zone, add a LMN_CONFIG.pv.snowCoverNote entry.

### 8.4 Add a new efficiency scenario

Status: To be tested (written from the code, not rehearsed)

- Upstream simulation: Define and simulate the new efficiency scenario rung upstream across all archetypes in EnergyPlus, and generate the master campaign outputs before creating configuration entries.

- Import through 8.1.

- Add its label to LMN_CONFIG.eemLabels and its long form to eemDetails.

- Add its campaign tag to LMN_CONFIG.eemCampaignTags, so the rung stays traceable.

- Add the rung to the Layer 2 selection screen.

### 8.5 Add a new technology option

Status: To be tested (written from the code, not rehearsed)

One file is edited, and there is no HTML step. Since the fix of 2026-09-25, the Layer 2 generation cards are built by renderGenerationCards() in js/energy-selection.js from LMN_CONFIG.selectionLabels.generation in js/config.js. layer2_energy_selection.html carries no hand written cards for this group any more; editing js/config.js alone puts the card on the screen.

| File | What to change |
| --- | --- |
| Content/Images_Layer2_EnergyGeneration/ | The icon, if the option needs a new one |
| js/config.js | One entry in LMN_CONFIG.selectionLabels.generation |

- Icon. Put the icon in Content/Images_Layer2_EnergyGeneration/, or reuse an existing one. Check the file name against the real directory listing, with exact case: GitHub Pages is case sensitive.

- Entry, js/config.js. Add the key to LMN_CONFIG.selectionLabels.generation, next to the existing entries (biomass, wind, geothermal). Fields:

  - label (required): the display text.

  - image (required): the explicit icon path.

  - popover (optional): { title, text }, shown by the card’s info icon.

  - statusId (optional): id of a hidden status <span> that other code toggles at runtime; omit it for a card whose status never changes.

  - unavailable (optional): true disables the card, adds data-permanently-disabled, and shows the status text LMN_CONFIG.availability.notModelledLabel (“Not modelled yet”).

  - alt (optional): image alt text; defaults to label when omitted.

- "new_key": { label: "New Technology", image: "Content/Images_Layer2_EnergyGeneration/new_key.png", unavailable: true }

- The sidebar and the finish-design summary read the label and the icon from here too.

- If no quantitative model exists for the option, set unavailable: true as above. Do not ship an option that accepts a click and changes no number: that is what facade solar did on all 35 neighbourhoods before it was restricted.

- Cache stamp. Because js/config.js changed, bump the ?v=N stamp on every page in the same commit (section 10.2).

- Check. Serve the site (section 10.1), open the Layer 2 selection page and confirm the card renders correctly (enabled, or greyed with “Not modelled yet” and unreachable by click or Tab if unavailable).

### 8.6 Update an image or a 3D model

Replace the file, keeping the filename byte identical including case, and bump ?v= if a script references it. If the filename must change, change it in LMN_CONFIG.selectionLabels in the same commit.

### 8.7 Update an assumption

Maintainers must check section 15.0 first and decide which of the three classes the assumption belongs to: simulation, post-processing, or display and configuration. The three are changed in three different places.

#### Simulation assumption (input to EnergyPlus)

If the assumption was an input to EnergyPlus (such as envelope U-values, heat pump COP, infiltration rates, weather files, schedules, ground boundary conditions, or code vintages), it cannot be changed in js/config.js or in this repository. Modifying a simulation assumption requires modifying the upstream model in idf_reader and executing the full update pipeline: 1. Modify the upstream model in idf_reader. 2. Re-run the EnergyPlus simulation campaign. 3. Re-export the campaign master CSV into Templates/<date>/. 4. Run the pipeline scripts (convert_master_csv.py, patch_data_js.py) to generate updated js/data.js. 5. Verify data integrity with test scripts (check_keys.py, test_data_flow.py, stage verifiers). 6. Bump ?v= cache parameter (section 11.1) and publish (section 11).

Follow the complete procedure in section 8.1.

#### Post-processing assumption (input to an offline calculation, not to EnergyPlus)

If the assumption is an EV or vehicle to grid parameter, a landscape solar chain constant, or the panel efficiency behind a shipped solar figure, editing js/config.js will not change the published number. The number was produced by a calculation outside the page and transcribed into js/data.js. The change is:

- Edit the calculation source: Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py for mobility, the landscape chain for Layer 4, the upstream injector for solar.

- Re-run it. For Layer 3 and Layer 4 that is generate_ev_lpv_data.py, which rewrites Templates/NUS_EV.csv, Templates/NUs_LPV.csv and the EV_V2G_DATA and LPV_DATA blocks of js/data.js.

- Update the mirrored value in js/config.js so the caption beside the result still matches it.

- Verify (section 10.2), bump ?v= (section 11.1), publish.

#### Display and configuration assumption (changes no published number)

If the assumption is a floor area basis label, a unit string, the status vocabulary, a note, or the mirrored copy of a value from either class above:

Edit js/config.js. Nothing else. Add a comment saying what changed, the date, and which decision approved it, in the style of the comments already there. If the value also appears literally in a page or a script, that is a defect: move it here and have the page read it. Then bump ?v= on affected pages (section 11.1) and publish (section 11). Nothing a visitor reads as a result will move.

### 8.8 Update text, a table or a figure shown in the interface

Four kinds of thing appear on screen and they are changed in four different places. Decide which kind it is before editing anything. Editing the wrong copy is how one value came to be written in three files that then drifted apart, which was three separate defects in this round.

A label, a caption, a status word, a note, or a number quoted in prose. js/config.js, and nothing else. This is where euiLabel, euiBasisCaption, floorAreaBasisNote, eemLabels and eemDetails, selectionLabels, notModelledLabel and the solar notes live. The pages read them. A number must never be typed into a page, section 8.7.

A result table or a chart. These are not edited, because they are not written anywhere: the page’s script builds them at run time from js/data.js. To change what one says, either change the underlying result, section 8.1, or change the script that lays it out, js/energy.js, js/pv.js, js/lpv.js, js/ev-v2g-breakdown.js or js/finish-design.js. Never hand edit a value in js/data.js, section 8.1.

Fixed prose and a hand written table. These are in the page’s own HTML. documentation.html is the public methodology page and carries sections A to O with their own tables; index.html carries the landing text; each selection page carries its own instructions. Edit the HTML directly. If the passage contains a number the tool also computes, stop: move the number into js/config.js and have the page read it, otherwise the two copies will part company at the next import.

A figure, an icon, a 3D model, or a reference PDF. Content/, by the rule in section 8.6: the filename stays byte identical including case, or it changes in js/config.js in the same commit. The reference PDFs are under Content/References & Methodology/ and are linked from documentation.html.

Then bump ?v= on every stylesheet and script reference you touched, in every page that loads it, and publish, section 11. A stale stamp is why a corrected page can look uncorrected in a browser that has visited it before.

### 8.9 Add a layer or a module, or connect another tool

Status: To be tested (written from layer4_lpv_breakdown.html, not rehearsed)

The worked example is Comparison Mode, which was built this way and is held unpublished behind LMN_CONFIG.comparisonMode.published.

A module here is three things and no more: a page, a script, and the state it agrees to read. Nothing else in the system has to be told it exists.

- Decide what it reads. Every page reads the same three sources: the query string, neighbourhood and envelope; sessionStorage, which carries the selections made in the earlier layers; and the stored results, through js/data.js and js/config.js. A module that needs a number nobody has simulated is a simulation task first, section 8.1, not an interface task.

- Create the page by copying an existing breakdown page, for example layer4_lpv_breakdown.html. Keep its head block whole, and keep the four scripts in their order: config.js, data.js, sidebar.js, then the page’s own script. The order is not cosmetic; the page’s script assumes the other three have already run.

- Create the script under js/. It must stop rather than guess when no climate has been chosen, using LMN_CONFIG.noClimateSelected as the existing pages do. No page in this tool is permitted to supply a default climate on the user’s behalf, and the reason is recorded in js/sidebar.js.

- Wire the navigation. The back and next buttons are ordinary links in the page’s HTML, so add the link that reaches the new page from the page before it. If the module is to appear in the left sidebar, add its key to buildSidebar(currentLayer, mode) in js/sidebar.js, which is a chain of named layers rather than a loop over a list, so a new name has to be written in by hand.

- Gate it if it is not finished. Add a flag to js/config.js carrying the reason, in the form comparisonMode uses, and have the entry point read the flag. An unfinished module ships hidden or does not ship: an option that accepts a click and changes no number is the defect described in section 8.5.

- Document it. Add it to documentation.html if a stakeholder will meet it, and add its data path to section 5 of this guide, so its numbers stay traceable in the same way the existing ones are.

- Run every check in section 8.1, steps 5 to 7, then publish, section 11.

Connecting another tool or an external data source. The tool is static, HTML, CSS and JavaScript served by GitHub Pages, with no server and no build step, section 3. There is nothing on the server side from which to call anything. Two routes are therefore open, and only the first has been used:

- Through the pipeline, which is the supported route. Have the other tool write a CSV, place it under Templates/ with its campaign date, and import it by section 8.1 so that its output becomes part of js/data.js like every other number. It then inherits the key checks, the data flow test and the traceability of section 5.

- From the browser, which has not been done. A module’s own script can call an external service directly, but that introduces a network dependency, a failure mode on pages that currently have none, and a cross origin requirement, and it puts a displayed number outside the provenance chain of section 6.0. It should be decided and recorded in this guide before it is written, not discovered afterwards in a script.

### 8.10 Interface design

Status: To be tested (written from codebase audit)

Where colours, fonts, cards/buttons, layout, charts and other visual elements are controlled.

The visual design system is centralised in css/styles.css using CSS custom properties (:root tokens). Adjusting the interface requires modifying these tokens and component classes rather than writing ad-hoc inline styles.

| Interface element | Where it is controlled | What must be done |
| --- | --- | --- |
| Colours | css/styles.css, the :root block at the top: --bg-primary, --text-primary, --accent-gold, --accent-amber, --accent-burnt, --accent-teal, --accent-sage, the banner tokens (--banner-*), and the five parameter button triples (--usage-*, --context-*, --diversity-*, --density-*, --layout-*) | Change the token, not the individual CSS rule. A colour written into a rule instead of a token is a defect |
| Fonts | --font-primary (‘Inter’), --font-display (‘Outfit’), --font-body (aliased to primary). Both faces are vendored in the repository under vendor/fonts/, not loaded from Google Fonts or a CDN | Replacing a typeface requires replacing the vendored .woff2 files in vendor/fonts/ and updating the @font-face blocks at the top of css/styles.css, in addition to updating the token |
| Cards and buttons | The class families in css/styles.css structured by component:- Selection cards: .usage-card, .context-card, .density-card, .layout-card, .concept-card, .envelope-card- Result and summary cards: .consumption-card, .generation-card, .demand-card, .load-card, .mobility-card, .summary-card- Layer 4 landscape PV cards: .lpv-card--gi, .lpv-card--lpv, .lpv-card--neighbourhood, .lpv-card--empty, .lpv-header-card- Layer 3 header cards: .ev-header-card- Card status badges: .card-status, .card-status--preliminary- Parameter buttons: .parameter-buttons, .ev-parameter-buttons, .lpv-parameter-buttons- Navigation buttons: .nav-buttons, .next-button, .back-button- Selection and action buttons: .nu-select-btn, .nu-action-btn, .envelope-tier-btn, .toggle-btn, .ev-toggle-btn, .lpv-toggle-btn, .initial-btn- Modal buttons: .modal-buttons, .confirm-btn, .cancel-btn, .submit-btn- Comparison mode: .comparison-mode-btn- Base cards and callouts: .card, .info-box, .info-box--caution | Naming convention: card classes are named after the thing they show, and modifiers use the double-dash form (e.g. .lpv-card--gi, .card-status--preliminary). Apply or extend existing class families; do not inline styles on HTML elements |
| Layout | The page’s own HTML file, plus the navigation sidebar, which is generated by buildSidebar() in js/sidebar.js | The sidebar is the one layout element that is not in the page HTML; modify buildSidebar() to alter sidebar links or structure across all pages |
| Charts | Bar and legend colours come from ENERGY_COLORS, which lives in js/config.js. Chart geometry and layout are in js/energy.js and js/comparison.js | To change a chart colour: edit ENERGY_COLORS in js/config.js. This was the documented exception until 2026-09-09, when the block moved out of the generated file. Before the move it sat in js/data.js, where a full regeneration would have lost the edit; routine runs did not, because Templates/scripts/patch_data_js.py patches only NEIGHBOURHOODS, PV_GENERATION_DATA and ENVELOPE_ENERGY_DATA and appends to BUILDING_IMAGES. The exception is closed |
| Images and 3D models | Content/ directory | Follow the asset update rules in section 8.6. Match filenames byte for byte |
| After any change | Bump ?v= on all affected HTML pages (section 11.1) | Every interface change concludes by cache-busting affected pages and publishing (section 11) |

#### Two critical rules for interface assets

Two operational rules must be observed to prevent defects:

- Case sensitivity: Local development is typically on Windows (case-insensitive file system), whereas GitHub Pages serves from Linux (case-sensitive). A file reference that differs only in case works locally but fails with a silent 404 live on GitHub Pages because <img> tags hide themselves on error. Always match file paths and extensions byte-for-byte in exact case.

- Never build a path or filename by interpolating a display label: Never construct an asset path (e.g. icon images) dynamically from a human-readable display string. Display labels change for presentation, which silently breaks asset loading. Always define and look up explicit, hardcoded asset paths in LMN_CONFIG.selectionLabels.

#### Worked example: the terminology chip

Added 2026-08-31 at Dr. Hachem-Vermette’s request, replacing four definition paragraphs at the top of three pages. It is the reference pattern for any future disclosure that should be available without occupying the page.

| Part | Where | What it does |
| --- | --- | --- |
| The content | LMN_CONFIG.abbreviations, js/config.js | Four terms, each a short, a full and a note. The only place they are defined. No page carries a copy |
| The chip | LMN_CONFIG.termChipHtml(short), js/config.js | Returns the short form followed by a real <button> and a hidden panel holding the full name and the note |
| The legend | LMN_CONFIG.terminologyLegendHtml(), js/config.js | One line, the cumulative ladder rule, then the four terms as chips |
| The mount | .js-terminology-legend, three pages | The pages carry an empty slot and no definition text. layer2_energy_selection.html, layer2_energy_breakdown.html, layer4_finish_design.html |
| The styling | .term-chip, .term-info-btn, .term-info-icon, .term-def-panel, css/styles.css | Tokens only: --bg-secondary, --border-subtle, --text-primary, --text-secondary, --accent-teal, --bg-card, --radius-sm, --shadow-card, --transition-fast |

Four rules the pattern follows, and a new one should follow them too.

- A real <button>, never a <span> with a click handler. It carries aria-label, aria-describedby and aria-expanded, is reachable by Tab, opens on Enter and Space, and shows a :focus-visible ring on --accent-teal.

- Hover and click, not hover alone. Hover does not exist on a phone. Click holds the panel open until a second click, an Escape, or a click elsewhere, and opening one panel closes any other.

- The mount stays inside try/catch. The verification suites run the pages against a stub DOM with no browser, so js/config.js must load in node. A page that cannot mount the legend must still render: the legend is an explanation, never a number.

- No new colour. Every rule uses an existing :root token.

After the change: bump ?v= on all fifteen root pages (section 11.1).

## 9. Dependencies

### 9.1 Front end

| Asset | Version | How loaded | Used by |
| --- | --- | --- | --- |
| Google model-viewer | 4.2.0 | Vendored, vendor/model-viewer/, 1,041,839 bytes, BSD-3-Clause | 3dviewer.html only |
| Inter and Outfit | Google Fonts, taken 2026-08-24 | Vendored, vendor/fonts/, four variable woff2, about 180 KB, SIL Open Font License 1.1 | css/styles.css, every page |

That is the complete list. There is no D3.js and no Chart.js in the current build. Both were used earlier and both are gone: the treemap was rewritten in plain DOM and CSS, and the hourly charts were removed as dead code. README.md says the same, corrected on 2026-09-09. Every chart, treemap and visual is drawn by the tool’s own code with plain DOM and CSS.

The tool now calls no external host at all, and that is recent. Until 2026-08-24 the 3D viewer loaded its library from ajax.googleapis.com, so all 35 models depended on a Google CDN and none drew on a restricted or offline network. The library was vendored the same day, on the owner’s decision, as DBG-044. The stylesheet’s font @import was the last remaining call and was closed as DBG-048; see Appendix F.

Both are pinned and neither updates itself. Each carries a README.md beside it recording where it came from, on what date, under what licence, and what updating it involves. The version is yours to move, which is the cost of vendoring and the reason it was a decision rather than a maintenance fix.

One thing to watch on the 3D viewer. The bundle names gstatic.com internally, for the Draco and KTX2 decoders. None of the 35 models needs them: all 35 were checked for KHR_draco_mesh_compression and KHR_texture_basisu and none carries either, so the request never fires. A compressed model added later would bring that external call back.

### 9.2 Pipeline

| Item | Version |
| --- | --- |
| Python | 3.x. Standard library only: csv, json, os, re, sys |
| Node.js | Any current version, for the verification scripts only |

### 9.3 Simulation, outside this repository

Measured on the working machine, 2026-09-09. Nothing here is inferred from a requirements.txt, because the upstream project does not have one: these are the versions actually installed in the environment that produced the published campaign.

| Item | Version and location |
| --- | --- |
| Python | 3.14.3, CPython, in a uv managed virtual environment at idf_reader/.venv. Invoked in the upstream scripts as .venv/Scripts/python.exe |
| Python packages, required | eppy 0.5.69 (IDF object model), pandas 3.0.1, numpy 2.4.3, matplotlib 3.10.8, pydantic 2.12.5, pytest 9.0.2. The README’s own install line is pip install matplotlib numpy pandas eppy pytest pydantic. There is no requirements.txt, no environment.yml and no pyproject.toml in idf_reader; this table is the dependency record |
| EnergyPlus | 22.1.0 produced the campaign behind the published data. Three versions are installed and supported, at the fixed Windows paths C:\EnergyPlusV22-1-0, C:\EnergyPlusV23-1-0 and C:\EnergyPlusV24-2-0. Every IDF declares its own Version field and BEM_utils.config.get_ep_paths() routes the job to the matching executable and IDD. A neighbourhood whose merged models disagree on version is rejected with version_mismatch |
| Operating system | Windows. The EnergyPlus paths above are hard coded as Windows drive paths, and the runner resolves energyplus.exe. There is a documented alternative for batch work, the Concordia Speed cluster, which runs the same jobs through SLURM and Singularity, README section “Cluster / HPC Offload” |
| Environment variables | None required. One optional: BEM_MAX_PREP_WORKERS=N overrides the neighbourhood preparation worker count, which otherwise comes from MAX_NEIGHBOURHOOD_WORKERS = 8 in BEM_utils/config.py. Each preparation worker uses about 250 to 450 MB |
| Weather data | CWEC2020v2 for the Canadian arms, TMY3 for the withdrawn Buffalo arm. 33 files, tracked, in Content/WeatherFiles/ |
| Disk | The tracked repository is modest. A full neighbourhood campaign writes tens of gigabytes into 0_BEM_Setup/SimResults_neighbourhoods/; the existing campaign store is about 252 GB (section 13.2) |

The website pipeline shares none of this. The scripts in Templates/scripts/ use only the Python standard library, section 7. A maintainer who only updates the website needs no EnergyPlus and no virtual environment.

### 9.4 Running the upstream simulation from a clean machine

Written 2026-09-09, closeout item A2. This section exists because a new HQP must be able to reach a simulation result without asking anyone where anything is. It is the one part of this guide that is entirely about the other repository. Work through it in order the first time.

#### 9.4.1 Set the machine up

- Clone the simulation repository. git clone https://github.com/orcunkoraliseri/NEXA-desktop (named idf_reader until 2026-09-28; private, so accept the collaborator invitation first), or, once the transfer of section 14 is complete, the RHLab controlled remote, which is the same history.

- Install EnergyPlus 22.1.0, to the default location C:\EnergyPlusV22-1-0. Install 23.1 and 24.2 as well if you intend to run models that declare those versions. Do not install to a different path: the routing table in BEM_utils/config.py looks for these exact directories.

- Create the environment and install the packages, versions in section 9.3:

- python -m venv .venv
.venv\Scripts\activate
pip install matplotlib numpy pandas eppy pytest pydantic

- Check the install before running anything: python -c "from Content.neighbourhoods.neighbourhood_registry import NEIGHBOURHOODS; print(len(NEIGHBOURHOODS))", from the repository root. It loads the canonical registry, prints 35, and fails loudly if the clone is incomplete. It needs no EnergyPlus. Do not use tests/test_neighbourhood_registry.py as the install check, as earlier versions of this guide said: it is a full simulation run, not a quick test. Its code runs at import, so pytest starts EnergyPlus for every one of the 35 neighbourhoods in the registry, one after the other, for hours; it writes a run folder per neighbourhood under 0_BEM_Setup/SimResults_neighbourhoods/ and rewrites the tracked input model Content/neighbourhoods/onesquare_RS-I1.idf in place (restore it with git checkout -- Content/neighbourhoods/onesquare_RS-I1.idf). Found in the HQP trial, 2026-10-07, Appendix G2 item Z3.

#### 9.4.2 Find what you are going to run

Four registries between them answer “which model, which climate, which scenario”, and none of them is a personal note:

| What you need | Where it is |
| --- | --- |
| Which neighbourhoods exist, what buildings compose each one, how they are placed and rotated | Content/neighbourhoods/neighbourhood_registry.py, the canonical registry, about 35 neighbourhoods across the RC-, RS-, MU-, CC- and IC- families. Building assignments are defined here and nowhere else |
| Which models exist, and for which climate arm | Content/00.BaselineBuildings_NUs holds the prototype buildings. The merged neighbourhood models are foldered by arm: 00.Baseline_NUs_CAN_Z4, _Z5, _Z6, _Z7A, _Z7B, _Z8, plus 00.Baseline_NUs_CAN_MTL, 00.Baseline_NUs_CAN_MTL_1983 for the Quebec 1983 reference envelope, and 00.Neighbourhoods_US_ASHRAE for the withdrawn Buffalo arm. The folder is how you pick the climate arm; the weather file is picked separately and the two must agree. The pipeline menu prints each folder with its file count on start |
| Which weather files exist, and which climate each one is | Content/WeatherFiles/, 33 .epw files. The zone to file mapping this tool publishes is the table in section 6.12 |
| Which scenarios exist and what each one adds | The EEM Journal ladder, section 6.5 of this guide and README section “EEM Retrofit Framework”: EEM_J_DEFAULT, EEM_J_ENVELOPE, EEM_J_ENV_HVAC, EEM_J_ENV_HVAC_DHW, EEM_J_ENV_HVAC_DHW_EEM4, cumulative in that order |

#### 9.4.3 Run something, at four scales

Everything is driven from one prompt-driven entry point, python main_BEM.py, whose top menu has eleven options. The four scales the checklist asks for map onto it as follows. Start at scale 1 and do not start at scale 4.

| Scale | What it is | How |
| --- | --- | --- |
| 1. One building | A single prototype, one weather file | Option 1, or Option 6 with sub-option b0 for the five rung Journal ladder on one building. It asks for the dataset (US_ASHRAE, CAN_MTL, CAN_CLG), lists the IDFs in that folder, and runs the tags you pick |
| 2. One neighbourhood | The merged neighbourhood model, one climate, one scenario | Option 8, then the sub-menu: 0 simulates the merged IDF as is, b1 to b4 run one Journal rung. The EPW picker lists Content/WeatherFiles/ by number |
| 3. One neighbourhood, one climate, the whole ladder | Five rungs, one neighbourhood, one weather file | Option 8 with sub-option b0, the five simulation master. This is the smallest run that produces a complete comparable ladder, and it is the right size for a test |
| 4. A full campaign | Every saved neighbourhood, every rung | Option 9 with sub-option b0 (9i, the Journal v2 master, five simulations per neighbourhood) or d1 (9j, the LMN sixteen scenario factorial). This is days of compute, eight workers by default. The cluster path exists for exactly this, README section “Cluster / HPC Offload” |

Two behaviours worth knowing before the first run. The version of EnergyPlus is chosen by the model, not by you: each IDF’s Version field is read and routed. Neighbourhood runs use a different airflow mode from single buildings, MultizoneWithoutDistribution, for stability at that scale.

#### 9.4.4 Tell success from failure

Do not read the console to decide whether a batch worked. Every batch writes master_status.json into its run folder with every planned row marked pending before anything starts, then flips each row to PASS or FAIL as jobs finish.

“Run folder” means the batch folder the run creates, not a folder named Run. It is in the idf_reader clone, under 0_BEM_Setup/SimResults_neighbourhoods/, and is named from the menu option, the neighbourhood and a timestamp, for example 0_BEM_Setup/SimResults_neighbourhoods/option8_RS-I3_i_20260915_205718/. master_status.json and master_summary.csv sit at the top of that folder.

- Per job: the row’s state in master_status.json, with a failure reason if it failed. version_mismatch means mixed EnergyPlus versions in one merged neighbourhood.

- Per run: eplusout.err in the run folder. EnergyPlus reports n_severes and n_warnings, and both are carried through into the master CSV, so a run that completed with severe errors is visible without opening the folder.

- Per batch: master_summary.csv, rendered from the status file, one row per planned simulation.

- A killed batch is resumable. Re-run the same command, or rebuild the summary from the status file with regenerate_master_report.py <batch_root>. There is also recover_master_report_from_csvs.py for the case where the status file is lost but the run folders survive.

Output lands in 0_BEM_Setup/SimResults_neighbourhoods/<campaign>/<climate>/<neighbourhood>/<run>/, where <run> is {NU}_{EEM tag}_{TIMESTAMP} and holds the injected IDF, the results/ folder, eplusout.sql, eplustbl.csv and the modified IDFs. That folder is the only copy of the raw evidence, section 13.2.

#### 9.4.5 Produce and validate the master CSV

The campaign master CSV is the boundary between the two repositories: it is the last upstream artefact and the first input the website takes.

- The batch writes master_summary.csv into the campaign root. For the neighbourhood campaigns the consolidated file is LMN_full_NU_master.csv, at the root of SimResults_neighbourhoods/.

- Check it before it travels. Row count against the plan; n_severes zero on every row you intend to publish; idf_path and output_path present on every row, because they are the provenance trail; and the EUI columns within a plausible range for the climate.

- Copy it into the website repository as Templates/<date>/LMN_national_NU_master.csv, a new dated folder, never over an existing one, alongside the IAL, PV and archetype CSVs the campaign produced (carry forward any it did not re-run).

- Point the pipeline at it. Change DATA_VERSION in convert_master_csv.py to the new folder’s name.

- Run the website pipeline, section 7: convert_master_csv.py, then patch_data_js.py, then check_keys.py and test_data_flow.py. All four are runnable from any working directory.

- Verify and publish, sections 10.2 and 11.1.

#### 9.4.6 What this section does not cover

The rename pass of section 6.12 has not been run. Until it is, the file names you meet upstream follow the historical patterns, not the settled ones, and the recovery table in section 6.12 is how you read them. The archive of the raw campaign folders does not exist yet either, section 13.2. Both are open items on the 2026-09-09 closeout checklist, A5 to A8, and both are recorded there rather than being quietly left out of this runbook.

## 10. Local testing

### 10.1 Serving the site

Open a terminal in the repository root, the folder that holds index.html, and enter:

python -m http.server 8000

Then open http://localhost:8000/index.html in the browser. A static server is required. Over a file:// address the 3D models will not load, because of browser security rules on local files.

To stop the server, return to the same terminal and press Ctrl+C. The terminal stays blocked while the server runs, so use a second terminal for git or the pipeline scripts.

### 10.2 Checks before committing

- python Templates/scripts/check_keys.py

- python Templates/scripts/test_data_flow.py

- The stage verification scripts under docs_implementation/documentation-revisions/Results/, run with node. They load the real js/data.js and js/config.js, run the real page logic against a stub DOM, and assert against the data rather than against a stored snapshot.

- Walk one complete session by hand: Layer 1 through the final summary, on one neighbourhood and one climate.

There is no test framework and no continuous integration. Adding either would be a genuine improvement and is not blocked by anything.

The 21 recurring failures, out of 706 checks. A22, closed 2026-09-09. The line-ending assertions were the reason ten of them failed on every fresh checkout: each asserted one fixed convention, CRLF or LF, while .gitattributes normalises the working tree, so they could never pass on any machine. They now test for mixed endings inside a single file, which is the defect that actually matters, and eight of the ten pass. The remaining 21 are not line-ending noise and should not be read as a baseline to ignore:

- All 21 are checks that outlived what they asserted, written against interface details from sessions 16, 17, 20 and stages 4 and 6 to 9 that later changed by decision. None of them is a data check, and no data check fails.

- The two that were a real defect are gone. js/pv.js mixed CRLF and bare LF, 621 against 6, and was normalised to one convention later the same day, DBG-051. Those two checks now pass, which is the whole of the difference between 683 and 685.

Measured 2026-09-09, after that change: 685 pass, 21 fail, 706 total. Anyone rerunning the suites should compare against those three numbers and treat any new failure as real.

### 10.3 Known differences between local and live

Case sensitivity is the recurring trap, and it has already caused production only failures. Development is on Windows, which ignores case. GitHub Pages is Linux, which does not. A path built by interpolating a display label into a filename works locally and returns a 404 live. That is exactly what produced the broken icons reported on the live site and not reproducible on the developer machine.

Containment now in place: every icon path is an explicit string in LMN_CONFIG.selectionLabels, sitting beside its label as an independent field so the two cannot drift, and each is asserted against the real directory listing by a check that walks the listing rather than asking a case insensitive filesystem. The silent onerror image hiding, which made every such failure invisible, was replaced by one console warning naming the file and a neutral grey plate.

Second difference: caching. A local server sends fresh files. GitHub Pages plus a browser will hold the old ones. Always test a deploy in a private window or with the cache disabled.

## 11. Deployment

### 11.1 Publish an update

git add -u                 # NOT -A. See the two warnings below.
git status                 # read every line of it before committing
git commit -m "…"
git push https://github.com/CarolineHVermette/NEXA-Web.git main

GitHub Pages redeploys in one to two minutes.

Live deployment target: the live site is deployed from the supervisor-controlled repository github.com/CarolineHVermette/NEXA-Web, at https://carolinehvermette.github.io/NEXA-Web/, since 2026-09-22 (D1, section 14.4 T7, completed).

Two warnings, corrected on 2026-08-24 after the first real release.

- Never git add -A here. The working tree normally carries deletions and untracked work that are not meant to go out in the same commit, and -A sweeps all of them in silently. On the 2026-08-24 release the 24 site files were staged by name for exactly that reason, so that 28 pending documentation deletions could not travel with them.

- origin has two push URLs, the site repository and the mirror. A bare git push writes to both. That is sometimes what you want. Push by URL when it is not.

### 11.2 Before pushing

- Bump the ?v= parameter on every script and stylesheet tag if any script, stylesheet or data file changed. If you forget this, the deploy will appear to do nothing. It is at ?v=16 on the published site of 2026-08-24, on 65 tags across 15 pages. Bump all of them together: two suites assert that no page is left on an older stamp.

- Confirm .nojekyll is still present at the repository root.

- Confirm no internal working folder is being published that should not be, section 3.

- Confirm no token, password or key is in any tracked file.

### 11.3 Verify the deployment

- Open the live URL in a private window.

- Check the version and campaign stamp in js/config.js by viewing the file directly on the live site.

- Walk one complete session.

- Open the browser console and confirm no 404 and no icon warning. This is the check that catches case sensitivity failures, and it is the only one that does.

### 11.4 Rollback

git revert the commit and push. GitHub Pages redeploys the previous state. Do not use a force push to roll back: it destroys the history that makes it possible to see what the live site was showing on a given date.

### 11.5 Tagging convention

Status: Waiting for CHV (joint item, after CHV’s trips, her email of 2026-08-30; tracked as D3).

Not yet established. The recommendation is v<tool version>-<date>, for example v0.9.0-2026-08-14, tagged at each publish, so that a released state can be recovered by name and a report can name the exact state it describes.

## 12. Known issues, workarounds and unfinished features

Sourced from docs_implementation/documentation-revisions/Debugs/DEBUG-REGISTER.md and this guide’s own assumptions register, section 15. The classes below were built per CHV’s instruction (A24) to separate real open defects, accepted V1 limitations, future research items and user choices, rather than mix them in one table. They were built from the existing record, and then confirmed against a fresh real-browser pass: A23 was run on 2026-09-09 and is recorded in Appendix F2. That pass added three entries to 12.1, DBG-052, DBG-053 and DBG-054, and corrected none of the others. Two of those three, and one older entry, closed on 2026-09-09: DBG-052, DBG-053 and DBG-051 are shown below as closed rather than deleted, so that a reader holding the earlier version can see what became of them. DBG-054 was half closed on 2026-09-10, when the missing simulations finished and their results were imported: it is repaired on the standard building and stands as written on the high performance building, which was never simulated for this case.

### 12.0 Open questions, in plain words

Three questions were asked in this round, and all three are answered. They are written here without technical wording so that they can be answered without reading the rest of the section.

- The Thermal Load button in Layer 2 did not change any result. Answered on 2026-09-10: the data was added, for the standard building. The button was asking for a column that had never been simulated. The simulations were commissioned on 2026-09-10 and finished the same day, 210 runs, all 35 neighbourhoods across the six climate zones, and the results were imported into js/data.js and checked against the source file value by value. The button now changes the numbers on the standard building in every published climate, and it still does not on the high performance building, because the simulations were run against the standard archetypes only. Half of what a visitor can reach is repaired and half is not, 175 combinations of 350. DBG-054 in 12.1 is the same account with the measurements attached, and it stays open for that reason.

- What should be bought for the archive? Answered at the meeting of 2026-09-10. One terabyte for each project, at a ceiling of CAD 300 to 400, on a portable SSD that Mac and Windows both read and write. Two shop links are in the covering letter. The earlier recommendation, one 4 TB portable drive and two 8 TB desktop drives, is withdrawn. Nothing can be archived or renamed until the drive arrives, which is checklist items A5 to A8.

- Comparison Mode works but is hidden from visitors. Answered on 2026-09-10. It goes offline rather than to review, and it is kept in the repository so that it can come back. comparison.html and js/comparison.js stay committed and nothing is deleted; the two entry points are now commented out as well, the anchor in layer4_finish_design.html and the block in js/finish-design.js, with LMN_CONFIG.comparisonMode.published left false as the record of the decision. To publish it later: uncomment both, set the flag to true, and bump the ?v= stamp on every page in the same commit. That is checklist item A27.

### 12.1 Real open defects

Items still requiring a code, credential or upstream fix.

| Ref | Issue | Priority | State |
| --- | --- | --- | --- |
| DBG-001 | A GitHub personal access token was embedded in the remote URLs of the working copy. Anyone with a copy of that working directory held a live credential, for both repositories, because the same token stood in all three URLs | P0, security | 🟨 In progress. Half closed 2026-08-24. The credential is out of the configuration: all four URLs in the website working copy now read https://github.com/... with nothing before the host, and the simulation repository was already clean. A history search confirms the token was never committed to a tracked file, so it did not go public with the repository. What remains is the revoke on GitHub, and it is Koral’s alone: a credential that has sat in a config file for months must be assumed seen. The token pushed on 2026-08-24, so it is the one showing last used within the last day. See section 14.5 |
| DBG-004 | The silent climate fallback, last seen in 16 sites across 8 files | P2, downgraded | ✅ Closed 2026-09-09, A15. Removed, not disclosed. The last 16 sites built the next page’s link rather than a number, and each now carries an empty envelope where it carried necb-2017, so the page it opens states the refusal. Two things settled it: the literal is not a key of ENVELOPE_ENERGY_DATA at all, so it never selected Montreal, it selected nothing; and the removal was measured in a real browser rather than assumed, 7 built links on six climate-less pages, 0 naming a climate, and the same links still carrying necb-z6 when one is chosen |
| DBG-051 | js/pv.js mixed line endings within itself, 621 CRLF against 6 bare LF. Found 2026-09-09 by the rewritten verification suites, and it was the only real failure among the 23 | P3 | ✅ Closed 2026-09-09. Normalised to 627 bare LF as a side effect of the A15 edit. The fear that it would bury the file history did not apply: core.autocrlf already keeps LF in the repository blob, so the recorded diff for the file is the A15 lines alone. Its two suite checks now pass |
| DBG-052 | comparison.html loads a charting library that was not in the repository, so the request returned 404, Chart was never defined, and both charts on the page were blank at every pixel. The same tag was the page’s only local reference with no ?v= stamp, and the page was the only one of fifteen with no icon link. Found 2026-09-09 by the A23 pass | P2 | ✅ Closed 2026-09-09. Chart.js 4.4.3 vendored to vendor/chartjs/, with a README stating version, licence and provenance, exactly as model-viewer and the two fonts are vendored; no CDN call was added. The tag now names the vendored file and carries the stamp, and the missing icon link was added with it. Measured in a real browser: 200 on the request, typeof Chart is function, no failed request and no external host on the page, and both canvases draw where they were blank. Comparison Mode itself is still gated and unpublished under A27 |
| DBG-053 | The seven card information icons on the option cards could be reached by keyboard and could not be opened by keyboard. Each is a <span role="button" tabindex="0"> whose only opening handler is a click listener. Verified in both directions with real key events: they open on a mouse click, not on Enter or Space, while every popover built as a real <button> opens on Enter. Found 2026-09-09 by the A23 pass | P3 | ✅ Closed 2026-09-09. A keydown branch was added beside the existing click branch in js/config.js, so Enter, Space and the legacy Spacebar key name open the focused icon and the whole behaviour stays in one handler. Escape still closes. The ?v= stamp moved to 26 on all 15 pages and 66 references in the same edit. Measured with real key events: 7 of 7 open on Enter, 7 of 7 on Space, 7 of 7 still take focus, and the mouse path is unchanged |
| DBG-054 | The Thermal Load card in Layer 2 was enabled on every neighbourhood and every climate and changed no number. Both resolvers asked for an IAL column, js/energy.js line 566 and js/config.js line 1707, and no cell in js/data.js carried one: 17 envelope keys, 595 cells, IAL on 0 of them. Both fell back to the baseline without saying so. Measured on 70 of 70 rows, the intensity shown after clicking the card equals the stored DEFAULT total, the same number the page shows with no card clicked, and the end use slices are identical to the decimal. Meanwhile the assumptions box prints “Measures applied: Thermal Load” and “Directly simulated”. Found 2026-09-09 by the A23 pass | P1 | 🟨 Half closed 2026-09-10. The data import was done and it covers the standard building only. The simulations the column needed were commissioned and finished on 2026-09-10: 210 runs, 35 neighbourhoods across the six NECB climate zones, zero EnergyPlus Fatals, and they were imported into js/data.js as an IAL block on each of those 210 cells. Every one of the 210 was read back out of the loaded file and checked against the source CSV field by field, 8 fields on each of 210 rows and 0 mismatches, with no all-zero row and no negative residual. Both resolvers now return IAL on those cells instead of falling back. What remains is the other arm: the campaign was run against the standard archetypes, so the high performance building has no IAL column and the card still falls back there without saying so. Measured across what a visitor can actually reach, the five published climates on both arms: 350 combinations, 175 now answer with the simulated ideal load and 175 still fall back, all 175 of them on high-performance-z4, -z5, -z6, -z7a and -z7b. The way out is the same as before and now applies to one arm rather than two: run the campaign against the high performance archetypes and import it the same way, or say so on the page. Published 2026-09-10, commit 5a956fb, and checked against the deployed server rather than the working copy: a real headless Chrome driven over the DevTools protocol through the live pages, 30 cases, 30 passing, no console error and no failed request. On the standard arm the intensity shown after clicking the card changed on 25 of 25 and equalled the stored ideal load total to the decimal on 25 of 25, across all five published climates. On the high performance arm it did not change on 5 of 5 and the assumptions box still printed “Measures applied: Thermal Load” beside the HPerf scenario, which is the half that stands open, measured rather than assumed. See Appendix F2 |
| DBG-055 | The vehicle to grid discharge field was a confirmed naming and unit defect: dischargeCapacity in js/data.js holds a daily energy, and the interface printed “10 kW / day,” which is not a valid unit. Confirmed in section 15.3 | P2 | ✅ Closed 2026-09-25, V1 wrap-up, CHV item 3 (WP3). Fixed 2026-08-12: js/config.js (RT01 terminology block, lines 1062 to 1108), layer3_ev_v2g_mobility_output.html and js/ev-v2g-breakdown.js now show “Daily V2G export per participating EV” in kWh/(EV·day). See Debugs/DEBUG-REGISTER.md DBG-055 |

### 12.2 Accepted V1 limitations

Known shortfalls that stand for this release, each with the decision that closed it.

| Ref | Limitation | Decision |
| --- | --- | --- |
| DBG-005 | Two conflicting solar values existed per neighbourhood upstream | ✅ Closed for V1, 2026-09-09. Website side resolved: the non-reproducing field was deleted and the authoritative source named. CHV’s decision E4: the 23 per cent panel efficiency is accepted for V1 on the basis of the traced Tier 3 injector that produced the published outputs, and the retired 18.65 and 20 per cent tracks are kept as provenance and history, not as competing active assumptions |
| DBG-019 | Rooftop solar is optimistic by 10 to 25 per cent in the three coldest zones because snow is not modelled | ✅ Closed for V1, 2026-09-09. CHV’s decision E5: do not re-run the V1 campaign for snow alone. The published numbers stand and the cold-climate optimism is disclosed on the page |
| DBG-028 | Five compact residential archetypes carry US prototype results in five NECB climates | ✅ Closed for V1, 2026-09-09 (open upstream, mitigated in the website). 25 pairs withheld, declared once, filtered from the Layer 1 table, explained in a grouped box, and gated across the four pages that could otherwise print one of their numbers. Nothing was deleted from js/data.js. CHV’s decision E6: no new campaign now, the combinations stay withheld and stay in the research record |
| DBG-027 | One mixed use neighbourhood has no baseline in Zone 7B | ✅ Completed. Resolved in website: fabricated baseline deleted, combination withheld with explanation |
| Flag f1, section 15.3 | The 90 per cent V2G discharge efficiency is displayed but is not applied by the export equation | ✅ Closed 2026-09-09. CHV’s decision E2: not claimed as the paper’s value, stated as a V1 limitation, not redesigned now |
| Flag f2, section 15.3 | The 50 per cent stationary storage share is the only unsourced constant in the Layer 3 chain | ✅ Closed 2026-09-09. CHV’s decision E3: retained provisionally because it is the paper’s own figure, stated as an assumption rather than a validated value; the resulting storage-loss figure is Preliminary |
| Facade solar, section 12.3 | Restricted to the nine Montreal neighbourhoods holding one of the three building types it was tested on, labelled Preliminary, excluded from the neighbourhood balance | Built and shown, deliberately limited. The restriction is asserted by an automated check |

### 12.3 Future research items

Work that requires new simulation, calculation or validation, deferred beyond V1.

| Item | Note |
| --- | --- |
| V2G methodology, efficiencies and loss assumptions | Per Dr. Hachem-Vermette, 2026-08-30 (CHV’s decision E1): the whole V2G module stays labelled Preliminary; the methodology requires further development and validation in future work |
| Explicit snow modelling for rooftop PV | Follows DBG-019; not undertaken for V1 |
| Upstream reconciliation of the retired 18.65 and 20 per cent solar tracks against the 23 per cent authoritative source | Follows DBG-005; not a release blocker |
| Re-simulation of the five compact residential archetypes on a Canadian rather than US prototype | Follows DBG-028; only if required, no new campaign scheduled now |
| Buffalo / ASHRAE arm re-run under Canadian weather | CHV’s decision E8: kept as a research/reference arm; ideally re-run under Canadian weather |
| Biomass, wind, geothermal, solar thermal, PV thermal generation models | Visible, disabled, marked “Not modelled yet.” A generation model for each, and a simulation campaign to produce per neighbourhood results. None was in scope for V1 |
| Most green infrastructure options | Green roofs, vertical greening, roof and food gardens, linear greenery and green spaces have no energy method in this round. Landscape solar is the one Layer 4 option with a calculation behind it |
| Facade solar, extended | Extending it means simulating the other building types and climates |
| Hourly solar generation charts | Removed from the interface. The hourly data exists for 5 of the 35 neighbourhoods. Recoverable the day the other 30 are produced upstream |

### 12.4 User choices

Deliberate scope or presentation decisions, not technical shortfalls.

| Item | Decision |
| --- | --- |
| DBG-009, landscape solar identical for all 35 neighbourhoods | ✅ Completed. Not a defect: the same land allocation is applied to every neighbourhood by decision, the uniformity is stated on the page, and the magnitude was corrected and closed as DBG-014 |
| DBG-020, the landing page’s second pathway | ✅ Completed. Closed 2026-08-24. “New Neighborhood” is disabled, aria-disabled, unreachable by keyboard, labelled Not modelled yet. CHV’s decision E11, 2026-09-09: the pathway stays visible, disabled and labelled in V1; not a handover blocker |
| Buffalo / ASHRAE arm | CHV’s decision E8: keep only as a research/reference arm, do not present it publicly as a Canadian public climate in V1 |
| Chisasibi / Zone 8 | CHV’s decision E7: keep in the research database and technical documentation, do not expose quantitative public results in V1 |
| Saving or sharing a design | Not built. State is in sessionStorage and is lost when the tab closes. The scenario can already be exported as .json and printed, which is what the round asked for, so nothing is lost that the user cannot keep |

### 12.5 Workarounds in place

| Workaround | Why it exists |
| --- | --- |
| The “Other” end use block on the demand treemap | EnergyPlus reports 14 end use buckets; the tool charts 6. Before this, the breakdown page rebuilt the total from the six visible uses and disagreed with the final summary in 2,845 of 2,854 cells. The stored total is now rendered and the residual is shown as a seventh block rather than discarded |
| The Ratio of Performance is computed at render time | 2,845 stored derived values would have gone stale the moment a total or a solar figure changed, and three such changes were already scheduled. Removing the stored copy removed the obligation rather than meeting it |
| Landscape solar values in js/data.js are generated from the configuration chain | The page used to publish a figure 5.9 times its own three configuration rows. Generating the values means the table and the chain cannot drift again, and a verification script asserts they agree on all 35 |
| Explicit icon path strings | See section 10.3 |

## 13. Backup and recovery

Written as procedure. Not yet tested end to end. Testing it once is an acceptance criterion of this guide and is not met.

### 13.1 Master copies

| Item | Location | In the repository |
| --- | --- | --- |
| Website source, data, content | github.com/CarolineHVermette/NEXA-Web | Yes |
| Simulation methodology | docs_methodology/ | Yes, since 2026-08-14 |
| Simulation code, input models, raw outputs | github.com/orcunkoraliseri/NEXA-desktop (named idf_reader until 2026-09-28), mirrored to github.com/CarolineHVermette/NEXA-Desktop, both private | Yes. Corrected 2026-08-24, section 6.0 |
| The revision round record | docs_implementation/ | Yes |

### 13.2 What is in GitHub, what is only on the machine, and what would be lost

Written 2026-09-09 against CHV’s closeout checklist, item A4, and measured on the machine the same day. Earlier versions of this section and of section 6.12 disagreed with each other: one called the campaign run folders reproducible scratch, the other called them the only copy. Both statements were too broad. The four classes below are the accurate answer, and they are what the archive decision turns on.

|  | Class | What it is | Where it lives | Lost with the machine? |
| --- | --- | --- | --- | --- |
| 1 | Version-controlled inputs | The EnergyPlus toolkit, 4,655 input models, 33 weather files, the neighbourhood registry; and in the website repository the pipeline scripts, Templates/, Content/, js/ | idf_reader on main, pushed, with a second remote on CHV’s account. NEXA on main | No. Both are pushed |
| 2 | Validated and retained outputs | The 12,734 tracked files under outputs/, the per building results the master CSVs were built from; the campaign master CSV in Templates/<date>/; js/data.js itself | Tracked in the two repositories | No |
| 3 | Unique raw campaign folders | The neighbourhood run folders: for every run, the injected IDF, eplusout.sql, eplustbl.csv, the modified IDFs and the previews. Measured 2026-09-09: 63 top level folders, of which 22 carry the IAL tag of a different project, and about 5,623 run folders in total at campaign / climate / neighbourhood / run depth, about 252 GB. Section 6.12 quotes 27 campaigns and 3,169 run folders, which is a narrower count taken earlier; the scope of what is archived has to be stated before anything is copied | idf_reader/0_BEM_Setup/SimResults_neighbourhoods/, excluded by .gitignore line 60. Zero files tracked | Yes. This is the only copy |
| 4 | Reproducible scratch | Intermediate run space: tmp/, the other 0_BEM_Setup/SimResults* trees, staging for the cluster | Untracked, about 197 GB in tmp/ | Yes, and it does not matter |

What would actually be lost if the machine disappeared tonight. Not the capability: the toolkit, every input model, every weather file and the per building outputs are pushed, so every published number could be regenerated, corrected and extended. What would be lost is class 3, the raw evidence behind each published number: the actual run that produced it, its eplusout.sql, and the injected IDF the rooftop solar array area was parsed out of. Regenerating it means re-running the campaigns, days of compute, and the new folder names and timestamps would not match the 2,951 provenance paths recorded in the master CSV. The provenance trail, not the science, is what is one disk failure away.

This is the archive decision, and it is joint. It is the same question as the external drive CHV raised on 2026-09-09, and it is item A5 of her checklist. Nothing is renamed upstream until class 3 is archived and the archive verified.

### 13.3 Restore the live site

- git revert or git checkout the desired commit or tag.

- Push to main.

- Confirm GitHub Pages is enabled and pointing at main at the repository root.

- Confirm .nojekyll is present.

- Verify per section 11.3.

### 13.4 Restore from a clone

The repository is fully self contained for the website. A clone plus a static server reproduces the tool exactly. There is nothing to install and nothing to configure.

### 13.5 Backup of simulation models and raw outputs

In place, through version control. The toolkit code, the input models, the weather files and the per campaign outputs are tracked in the idf_reader repository and pushed to origin. A second remote already points at CHV’s own account, github.com/CarolineHVermette/LMN-Desktop, which is what an RHLab controlled copy means in practice.

✅ Done, 2026-08-24. That second remote was two commits behind on the morning of that day, 5a1ee23b against ee32e462, and was pushed the same afternoon and verified against the remote afterwards. The RHLab copy is now the copy that produced the published website. This was the action called T12.

Status: Waiting for CHV (joint item, after CHV’s trips, her email of 2026-08-30; tracked as D5, Appendix C row 13) for establishing a long-term RHLab-controlled archive independent of personal accounts.

What this does and does not prove. It exercised the backup in one direction: the copy is current and the push path works. Restoring from it has never been tried. Section 13 stays marked as tested once rather than tested end to end until someone clones the mirror and serves it.

Raw simulation campaign archive (SharePoint / Drive): Does not exist yet. No off-machine archive location has been created or agreed for 0_BEM_Setup/SimResults_neighbourhoods/, the 27 campaigns and 3,169 run folders from the published runs. This is open items A5 (archive the campaigns) and A6 (verify the archive) on the 2026-09-09 closeout checklist, and it is a precondition for the rename pass, section 6.12, which itself has not been run (A7, A8).

## 14. Accounts, access and ownership

No password, token or key appears in this document. It records which accounts and permissions are needed, not their secrets.

### 14.1 Accounts required

| Account | Needed for |
| --- | --- |
| GitHub, with write access to the repository | Every change, and every publish |
| GitHub, with admin on the repository | Enabling and configuring Pages, managing collaborators, transferring ownership |
| Access to the machine or archive holding the simulation project | Regenerating, correcting or extending any number |

Nothing else. There is no hosting account, no domain registrar, no database, no analytics service and no paid service of any kind.

### 14.2 Current state

| Item | Status |
| --- | --- |
| Destination repository | github.com/CarolineHVermette/NEXA-Web, created and configured as a remote, serves the live site since 2026-09-22 |
| Predecessor repository | github.com/orcunkoraliseri/NEXA-web, under a personal account, still a configured remote, no longer serves the site |
| CHV access | Granted |
| Live site | GitHub Pages, https://carolinehvermette.github.io/NEXA-Web/ |
| Master data copy | The repository, plus a personal machine for the simulation project |

### 14.3 Target state

| Item | Target |
| --- | --- |
| Repository location | RHLab or CHV controlled, which the destination repository satisfies |
| CHV role | Owner, able to manage access, settings, deployment and collaborators |
| Koral role | Not finalised, and deliberately so. CHV’s decision E12, 2026-09-09: the role is settled only after the ownership and deployment transfer is complete. Contributor or collaborator can be considered then. Ownership remains with CHV and RHLab, and Koral does not remain the system owner |
| Live site | Served from the RHLab controlled repository |
| Simulation project | Present in an RHLab controlled archive. Half done: the mirror on CHV’s account is current, an archive independent of either personal account is not agreed |

Status of handover items: Finalising owner transfer, role permissions and long-term archive is Waiting for CHV (joint items D5, D6, after CHV’s trips, her email of 2026-08-30).

### 14.4 Transfer checklist

| # | Step | Owner | Status | Note |
| --- | --- | --- | --- | --- |
| T1 | CHV creates a GitHub account with the Concordia email | CHV | ✅ Completed | Done |
| T2 | CHV given access to the repository | Koral | ✅ Completed | Done |
| T3 | Walk CHV through the repository live: structure, branches, data location, how the site is generated, tested and deployed | Koral | 🟦 Waiting for CHV | joint item, after CHV’s trips, her email of 2026-08-30 (D4; also closes Appendix C row 7) |
| T4 | Agree the final destination | Both | ✅ Completed | Done, CarolineHVermette/NEXA-Web |
| T5 | Destination created with CHV at owner level | CHV | ✅ Completed | Done |
| T6 | Content transferred | Koral | ✅ Completed | Done |
| T7 | GitHub Pages enabled on the destination and the live URL verified | Both | ✅ Completed | Done, 2026-09-22: https://carolinehvermette.github.io/NEXA-Web/ answers 200 (D1) |
| T8 | Every link updated to the new URL, in the documentation, the README and any published reference | Koral | ✅ Completed | Done, 2026-09-22, in this guide, the README and the trial packages (D2). Dated records keep the old URLs |
| T9 | Koral added as contributor or admin on the destination | CHV | 🟦 Waiting for CHV | joint item, after CHV’s trips, her email of 2026-08-30 (D6) |
| T10 | Confirm the tool builds and deploys with no dependency on a personal machine | Both | 🟦 Waiting for CHV | joint item, after CHV’s trips, her email of 2026-08-30 (D4) |
| T11 | Repeat the walkthrough until CHV is comfortable navigating and deploying independently | Both | 🟦 Waiting for CHV | joint item, after CHV’s trips, her email of 2026-08-30 (D4) |
| T12 | Bring CHV’s mirror of the simulation repository up to date | Koral | ✅ Completed | Done, 2026-08-24. Reduced first and then closed: the simulation project is not on a personal machine, it is the idf_reader repository, CHV already has a remote on it, and the two commits it was behind were pushed the same afternoon and verified against the remote. Section 6.0 |

### 14.5 Access and credential handling

The working copy’s remotes were configured with an embedded GitHub personal access token in the URL, in all three of them, so the one credential carried write access to the site repository and to the mirror alike. Anyone with a copy of that working directory, or anyone reading the git configuration on a shared machine, held it.

Steps 2 and 4 are done. Steps 1 and 3 are not, and they are the owner’s.

- ⬜ Revoke the token in GitHub, under developer settings, personal access tokens. This is the only step that actually closes the exposure, because a credential that has sat in a configuration file cannot be un-seen. The value is no longer on the machine, so identify it by last used: it pushed on 2026-08-24 and will show as used within the last day.

- ✅ The remotes are clean, 2026-08-24. All four URLs in the website working copy read https://github.com/... with nothing before the host, and the simulation repository was already clean.

- ⬜ Use the Git Credential Manager or an SSH key. The next push will ask for authentication and the credential manager will offer to remember it. Do not paste a token back into a remote URL — that is what created this item.

- ✅ Verified never committed. A history search across every tracked file for the standard GitHub personal access token prefixes returns nothing, so the credential did not go public with the repository. The exposure was local.

One thing did not change with the cleanup. origin still carries the mirror as a second push URL, so a bare git push still writes to both repositories. Only the credential was removed. Push by explicit URL, section 11.1.

Tracked as DBG-001, priority P0 security. The token value is not written anywhere in this guide, and must not be written into any document in this folder.

### 14.6 Handover materials

Written 2026-09-24, on a suggestion from the HQP trial. A new maintainer receives two repositories and a small set of documents. The files of each repository are listed in Appendix A: A.2 for NEXA-web and A.3 for idf_reader, the simulation repository. The documents are these:

| Document | What it is for | Where it is |
| --- | --- | --- |
| This guide | How the tool works and how to maintain it | NEXA-web, docs_implementation/documentation-revisions/Submission/NEXA_Handover-and-Maintenance-Guide_v3.4.md; also sent as .docx |
| Simulation Model Index | One row per simulation run behind the published tool | NEXA-web, docs_implementation/documentation-revisions/Submission/submission1/N-LENS_Simulation-Model-Index.csv; section 6.13 |
| Verification scripts | The checks run before a commit | NEXA-web, docs_implementation/documentation-revisions/Results/*.js; section 10.2 |
| Handover closeout and HQP trial checklist | Dr. Hachem-Vermette’s checklist: section A for Koral, section B for the trial preparation, section C for the trial tasks. The trial package cites its item numbers (B1, C1 and so on) | Sent by email as a .docx; not in either repository |
| HQP trial package | The trial tasks and the findings log | Sent by email as a .docx; not in either repository |
| Public methodology | The scientific basis shown to users | The site’s documentation.html |
| Simulation evidence | The origin of every published number | idf_reader/docs_DONE/docs_LMN_web/; section 6.0 |

## 15. Assumptions register

Every value the tool asserts, with where it is stored and where it came from. The authoritative copy is js/config.js, where each entry carries the same provenance as a comment. This table is that file, read in one place.

### 15.0 Three kinds of assumption

Before modifying any value, decide which of three classes it belongs to. The three are changed in different places, by different people, on different timescales, and getting the class wrong is the single most expensive mistake a maintainer can make here: an afternoon’s edit in js/config.js will not move a published number, and a campaign re-run to change a caption is a week wasted.

|  | 1. Simulation assumption | 2. Post-processing assumption | 3. Display and configuration assumption |
| --- | --- | --- | --- |
| What it is | A value that was an input to EnergyPlus | A value that is an input to an offline calculation which produces a published number without EnergyPlus | A value the tool shows, labels or formats, which produces no published number |
| Examples measured in this repository | Envelope U-values; the heat pump COP; infiltration; the weather file; schedules; the ground boundary method; the NECB or 1983 code arm | The six EV and vehicle to grid values in calculate_ev_scenarios.py; the landscape solar chain, 20,234 m2 x 0.20 x 0.10 x 0.200 kW/m2 x 1,280 kWh/kWp; the PV panel efficiency that produced the shipped generation | Unit strings; floor area basis labels; the status vocabulary; every note field in js/config.js; the mirrored copies of classes 1 and 2 that exist only to be printed beside a result |
| Where it is changed | The upstream model in idf_reader, not in this repository at all | The calculation source: Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py for mobility, the landscape chain for Layer 4, the upstream injector for solar | js/config.js, and nothing else |
| What must follow | Re-run the campaign, re-export, Templates/, pipeline, js/data.js, verify, publish (section 8.1) | Re-run that calculation, regenerate its block in js/data.js, verify, publish | Bump ?v=, publish (section 11.1) |
| How long | Days, and it changes published numbers | Hours, and it changes published numbers | Minutes, and it changes no number at all |

The trap is class 2, and it is worth one worked example. The landscape solar chain is written out in LMN_CONFIG.lpv in js/config.js, five constants with their arithmetic in a comment above them. Editing those five constants changes nothing a visitor reads as a result. The published figures are the strings already sitting in LPV_DATA in js/data.js, and js/config.js supplies only the specific yield caption and the two explanatory notes. To change the landscape solar output, the chain is recomputed and LPV_DATA regenerated. Mobility works the same way: calculate_ev_scenarios.py holds the six values, EV_V2G_DATA in js/data.js holds what is displayed.

Documentation mirrors in js/config.js: A simulation assumption may be mirrored into js/config.js as documentation, so that the tool can print it beside the result. The mirror is a caption or display label. Editing the mirror in js/config.js changes the caption and not one published kilowatt-hour. Changing the value itself is still the upstream, full simulation re-run path.

Worked example, three classes on one screen: the rooftop solar page shows the panel efficiency that produced the generation figure (class 2, changed upstream in the injector and re-exported), the envelope U-value of the buildings underneath it (class 1, changed only by re-running the campaign), and the words “Simulation-backed” and “kWh/m2 per year” (class 3, changed in js/config.js in minutes). All three look equally editable, and only one of them is.

### 15.1 Units and bases

| Assumption | Kind | Value | Stored in | Source or decision | Used by |
| --- | --- | --- | --- | --- | --- |
| Energy basis | Display | Site energy. EnergyPlus Total End Uses, electricity plus gas, no source multipliers | js/config.js units.energyBasis | Measured 2026-08-10 | Every intensity |
| Intensity floor area basis | Display | Heated and cooled floor area, EnergyPlus Net Conditioned Building Area | js/config.js units.intensityFloorAreaBasis | Measured: matches the conditioned area 35 of 35, the gross area 11 of 35 | Every intensity, every absolute total |
| Displayed floor area basis | Display | Total built area, EnergyPlus Total Building Area | js/config.js units.absoluteFloorAreaBasis | Same measurement | Display only |
| Intensity unit label | Display | kWh/m2 of heated and cooled floor area per year | js/config.js units.eui | Decision | Every page carrying an intensity |

### 15.2 Solar

Note on solar assumptions: The js/config.js entries below are documentation mirrors of inputs to the Tier 3 injector in idf_reader (see section 15.0 on documentation mirrors). Editing them in js/config.js changes only display labels, not simulation results.

| Assumption | Kind | Value | Stored in | Source |
| --- | --- | --- | --- | --- |
| Panel efficiency, both roof groups | Simulation | 0.230, labelled “23 % panel efficiency” | js/config.js pv.moduleEfficiency | BEM_utils/pv_tier3.py line 50, the Tier 3 injector that produced every published value |
| Active fraction, flat roof | Simulation | 1.0 on the rack | js/config.js pv.roofGroups.flat.activeFraction | BEM_utils/pv_tier3.py lines 617 to 623 |
| Active fraction, pitched roof | Simulation | 0.85 of the south roof face | js/config.js pv.roofGroups.pitched.activeFraction | BEM_utils/pv_tier3.py lines 51, 480 to 485, 651 |
| Retired, never used to produce a value | Simulation | 18.65 % cell efficiency behind a 0.80 fraction; 20 % module efficiency of aperture behind a 0.90 fraction | js/config.js pv.efficiencyNotes.retired | Tier 1 injector, retired 2026-05-29; and a literature reference |
| Ground coverage ratio, flat roof | Simulation | 0.40 | js/config.js pv.gcr | Same |
| Ground coverage ratio, pitched roof | Simulation | Does not apply | js/config.js pv.roofGroups.pitched.gcrApplies | Same |
| Array tilt, flat roof | Simulation | 45 degrees, facing south | js/config.js pv.roofGroups.flat.tiltLabel | Same |
| Array tilt, pitched roof | Simulation | Follows the roof pitch | js/config.js pv.roofGroups.pitched.tiltLabel | Same |
| DC to AC losses | Simulation | 14 % | js/config.js pv.dcToAcLosses | Same |
| Inverter efficiency | Simulation | 96 % | js/config.js pv.inverterEfficiency | Same |
| Authoritative solar field | Display | ENVELOPE_ENERGY_DATA[env][NU][scenario].pv | js/config.js pv.authoritativeSource | Measured: reproduces the simulation output 35 of 35 |
| Snow loss, Zones 7A, 7B, 8 | Display | 10 to 15 %, 15 to 20 %, 20 to 25 % optimistic | js/config.js pv.snowCoverNote | Upstream validation. Noted, not applied |
| Facade solar coverage | Simulation | Nine Montreal neighbourhoods, preliminary, excluded from totals | js/config.js facadePv | Three single building runs, Montreal only |

### 15.3 Mobility and vehicle to grid

Layer 3 figures are Preliminary in the maturity vocabulary of Appendix B (not a building energy simulation). On disk, the six calculation values are the default arguments of Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py. The equations there are numbered 12 to 18, and the source is identified, 2026-08-31: Hachem-Vermette, C. (2025), Designing energy-positive neighborhoods: a modular framework for integrated planning and policy guidance, Energy Reports 14, 4492 to 4507, section 2.4, equations 12 to 18. The match is exact, symbol for symbol, and equations 1 to 11 of the paper are the photovoltaic and heating chain. The earlier negative search is kept in docs_implementation/documentation-revisions/DeepResearchPrompts/responses/RT02_chv_ev_v2g_source_paper.md. The 15 kWh per EV daily demand carries a stated basis in Templates/NUS_EV.csv, a 200 km daily range, and section 2.4 of the paper states the same figure. As stated by Dr. Hachem-Vermette on 2026-08-30, the V2G methodology, efficiencies and losses will require further development and validation in future work.

Execution and data flow chain:

calculate_ev_scenarios.py  (the six default arguments)
  -> generate_ev_lpv_data.py
     -> rewrites Templates/NUS_EV.csv, Templates/NUs_LPV.csv,
        and the EV_V2G_DATA and LPV_DATA blocks of js/data.js

Changing an EV assumption means editing the default arguments in Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py and re-running generate_ev_lpv_data.py, not editing js/config.js and not editing js/data.js.

| Assumption | Kind | Value | Stored in | Note |
| --- | --- | --- | --- | --- |
| Vehicles per household | Post-processing | 1.5 | calculate_ev_scenarios.py (n_ev_per_household); lands in js/data.js EV_V2G_DATA[NU][scenario].evPenetrationRate | Default argument (Eq 12); Energy Reports 14 (2025) 4492 to 4507, Eq 12, and section 2.4 states 1.5 EVs per household |
| Daily charging demand per vehicle | Post-processing | 15 kWh | calculate_ev_scenarios.py (e_ev_per_day); lands in js/data.js ….dailyEnergyDemand | Default argument (Eq 12); Energy Reports 14 (2025), Eq 12. Basis in NUS_EV.csv is a 200 km daily range, and section 2.4 gives the same figure citing Dalla Chiara et al. (2019) |
| Charging efficiency | Post-processing | 90 % | calculate_ev_scenarios.py (eta_charging); lands in js/data.js ….chargingEfficiency | Default argument (Eq 12); Energy Reports 14 (2025), Eq 12, where it is the divisor grossing up demand. Section 2.4 states 90 per cent |
| Vehicle to grid participation | Post-processing | 50 % | calculate_ev_scenarios.py (p_v2g); lands in js/data.js ….v2gParticipationRate | Default argument (Eq 16); Energy Reports 14 (2025), Eq 16, where section 2.4.2 states 50 per cent. EV2 scenario only |
| Discharge energy per vehicle per day | Post-processing | 10 kWh | calculate_ev_scenarios.py (e_v2g_per_day); lands in js/data.js ….dischargeCapacity | Default argument (Eq 16); Energy Reports 14 (2025), Eq 16, where section 2.4.2 states 10 per participating vehicle per day and writes the unit as kW, which was the same defect. EV2 scenario only. The field name dischargeCapacity still holds a daily energy, but the interface unit string is fixed: DBG-055, closed 2026-09-25 (fixed 2026-08-12), js/config.js now labels it “Daily V2G export per participating EV” in kWh/(EV·day) |
| Discharge efficiency | Post-processing | 90 % | calculate_ev_scenarios.py (eta_battery); lands in js/data.js ….batteryEfficiency | Default argument (Eq 13); the paper uses it in Eq 13 but never gives it a value, so the 90 per cent is the code’s own. Displayed but not applied to exported energy, as in the paper’s Eq 17. Flag f1 closed 2026-09-09: not applied, stated as a V1 limitation, not claimed as the paper’s value |
| Stationary storage share | Post-processing | 0.5 | calculate_ev_scenarios.py line 31 | Unsourced; assumed 50% through stationary storage; only undocumented constant in Layer 3 chain; no literature convention sets a 50 per cent pass through; a candidate reading is a standby drain of about 0.83 kWh per vehicle per day, unconfirmed. Flag f2 closed 2026-09-09: retained provisionally as the paper’s own value, stated as an assumption, not validated |
| Storage loss | Post-processing | 5 % of total EV demand | Derived as 0.5 x (1 - battery efficiency); lands in js/data.js ….storageLoss as kWh/day | Derived calculation output, not an input (30 kWh/day for RC-R, 60 for RC-D) |
| Sign convention | Display | Positive means the grid must supply | js/config.js ev.positiveMeans | Stated in words on screen (ISO 52000-1 / ASHRAE 228) |
| Status | Display | Preliminary | js/config.js ev.preliminaryNote | Documented calculation chain, not a simulation (Appendix B) |

### 15.4 Landscape solar

| Assumption | Kind | Value | Stored in | Note |
| --- | --- | --- | --- | --- |
| Site area | Post-processing | 20,234 m2, a uniform 5 acres | js/config.js lpv.siteAreaM2 | Project decision, identical on every neighbourhood, stated on the page |
| Land allocation | Post-processing | 20 % of the site | js/config.js lpv.landAllocationFraction |  |
| Usable fraction | Post-processing | 10 % of the allocated land, in series | js/config.js lpv.usableFraction | Below the published 30 to 50 % range. Conservative, not wrong |
| Module power density | Post-processing | 200 W/m2 | js/config.js lpv.moduleWattsPerM2 | 400 W modules at 2 m2 |
| Specific yield | Post-processing | 1,280 kWh/kWp per year | js/config.js lpv.specificYieldKWhPerKWpYr |  |
| Resulting installed capacity | Post-processing | 80.9 kWp | Generated |  |
| Resulting generation | Post-processing | 103.6 MWh per year | Generated |  |
| Status | Display | Preliminary | js/config.js lpv.status | Not a simulation |

### 15.5 Climates and coverage

| Assumption | Kind | Value | Stored in |
| --- | --- | --- | --- |
| The five published climate arms and their representative cities | Display | Part I, section C | js/config.js climates |
| The two arms present in the data and not published, ASHRAE and NECB Zone 8 | Display | Section 6.3 | js/config.js withdrawnClimates |
| The 17 envelope keys and their display names | Display |  | js/config.js envelopeLabels |
| Withheld neighbourhood and climate pairs | Display | 26 pairs, with the reason for each | js/config.js dataGaps |
| Climate selection is mandatory | Display | true | js/config.js requireClimateSelection |

### 15.6 Vocabulary

| Term set | Kind | Values | Stored in |
| --- | --- | --- | --- |
| Maturity of a number | Display | Simulation-backed, Preliminary, In development, Not modelled yet | js/config.js statusTerms |
| A modelled but unpublishable result | Display | Not currently available | js/config.js availability |
| The six charted end uses | Display | Heating, Cooling, DHW, Lighting, Equipment, Fans and Pumps | js/config.js endUses |

### 15.7 Source publications

One published paper is a direct source of values used by the tool. It is listed here so that the assumptions above can be checked against it.

| Ref | Publication | What it sources | Where it is cited |
| --- | --- | --- | --- |
| R1 | Hachem-Vermette, C. (2025). Designing energy-positive neighborhoods: a modular framework for integrated planning and policy guidance. Energy Reports, 14, 4492 to 4507 | Section 2.4, equations 12 to 18: the whole Layer 3 electric vehicle and vehicle to grid chain, and the six default values it runs on. Equations 1 to 11 of the paper are the photovoltaic, district heating and heat pump chain, which is why the vehicle block starts at 12 | Sections 6.8 and 15.3, and the assumption table of 15.3 row by row |

Local copy: docs_implementation/documentation-revisions/Resources/1-s2.0-S2352484725006365-main.pdf. Identified on 2026-08-31 and verified equation by equation against Templates/Content_Layer3_Transportation/calculate_ev_scenarios.py. The match is exact, symbol for symbol, for all seven equations.

Two things the paper does not settle, and both are open questions for Dr. Hachem-Vermette. It writes the 50 per cent stationary storage share inside equation 13 without justifying it, and it never gives the battery efficiency a numeric value, so the 90 per cent in the code is the code’s own. Its equation 17 subtracts the vehicle to grid discharge with no efficiency term, which is what the tool does.

## Appendix A, glossary

### A.1 Terms

| Term | Meaning |
| --- | --- |
| NEXA | Neighbourhood Energy eXploration & Analysis, the tool |
| NU | Neighbourhood Unit, one of the 35 archetypes |
| EEM | Energy Efficiency Measure, the scenario rungs |
| EUI | Energy Use Intensity, annual energy per unit floor area |
| GFA | Gross Floor Area, here the total built area including unheated space |
| Conditioned area | Heated and cooled floor area, the denominator of every intensity |
| LPV | Landscape Photovoltaics |
| V2G | Vehicle to Grid |
| RoP | Ratio of Performance, annual on site solar generation divided by annual total demand, both as site energy. A published metric of the neighbourhood paper |
| NECB | National Energy Code of Canada for Buildings |
| NBC 9.36 | National Building Code of Canada, section 9.36, governing housing |
| BTAP | The NRCan Building Technology Assessment Platform, source of the Canadian occupancy and load schedules |
| GCR | Ground Coverage Ratio, a flat roof array quantity |
| IAL | Ideal Air Loads, the ideal thermal load option |
| PED | Positive Energy District |

### A.2 Folders and files

Where everything is in the repository, github.com/CarolineHVermette/NEXA-Web, top level first. Section 4 describes each one in full; this table is for finding it. The predecessor repository, github.com/orcunkoraliseri/NEXA-web, holds the same tree; it served the live site until 2026-09-22, see section 3.

| Path | What is in it | Detail |
| --- | --- | --- |
| index.html | Layer 0, the entry page | 4.1 |
| layer1_*.html to layer4_*.html | The eleven layer pages, selection then result, in the order a user walks them | 4.1 |
| layer4_finish_design.html | The final consolidated summary | 4.1 |
| documentation.html | The public methodology and references | 4.1 |
| 3dviewer.html | The standalone 3D viewer, opened from the archetype cards | 4.1 |
| comparison.html | Comparison mode. Present in the repository, not linked from the live navigation | 12 |
| js/ | Every page’s logic, one file per page, plus config.js, the shared constants, and data.js, generated | 4.2 |
| css/ | styles.css, the whole design system, and three historical backups | 4.3 |
| Content/ | Images, models and reference PDFs. Twenty five image folders | 4.4 |
| Content/Glb_Models/ | The 3D models the viewer loads | 4.4 |
| Content/IFC_Models/ | The BIM interchange models | 4.4 |
| Content/References & Methodology/ | The reference PDFs | 4.4 |
| Templates/ | The data pipeline: the per climate CSVs that are the import layer, and the dated source datasets | 4.5 |
| Templates/scripts/ | The four Python scripts that carry a campaign into js/data.js | 4.5, 7 |
| vendor/ | The libraries and fonts kept in the repository: Chart.js, model-viewer, Inter and Outfit. No page calls a CDN | 9.1 |
| README.md | The repository readme |  |
| GraphicalAbstract.md | The graphical abstract text |  |
| .nojekyll | Empty, and required. Without it GitHub Pages drops every folder whose name starts with an underscore | 3 |
| .gitattributes, .gitignore | Line endings, and what is kept out of the published repository | 10.2 |

One folder named in section 4.6 is deliberately not on GitHub. docs_implementation/documentation-revisions/ is listed in .gitignore and lives only on the working machine, because it holds the internal correspondence. Nothing on the site links to it, so nothing breaks. docs_methodology/ is tracked since 2026-10-07: it holds the methodology of NEXA-web and, in docs_methodology/HandoverDocument/, this guide and the complete trial package as .docx, the starting point for whoever takes the project over. Three exceptions have been tracked since 2026-09-17, so that a fresh clone carries what the HQP trial needs: this guide’s .md, the Simulation Model Index and the verification scripts in Results/ (section 14.6).

### A.3 Simulation repository, idf_reader

The files a maintainer needs in the simulation repository, github.com/orcunkoraliseri/NEXA-desktop, named idf_reader until 2026-09-28. Section 9.4 is the procedure; this table is for finding things.

| Path | What is in it | Detail |
| --- | --- | --- |
| README.md | Install line, the pipeline menu, the cluster offload | 9.4 |
| main_BEM.py | The single entry point for every simulation run, prompt driven | 9.4.3 |
| Content/neighbourhoods/neighbourhood_registry.py | The canonical registry: which buildings make each neighbourhood. Definitions only, no results | 9.4.2 |
| Content/00.BaselineBuildings_NUs/ | The prototype building models | 9.4.2 |
| Content/00.Baseline_NUs_CAN_*/, Content/00.Neighbourhoods_US_ASHRAE/ | The merged neighbourhood models, one folder per climate arm | 9.4.2 |
| Content/WeatherFiles/ | The weather files | 6.3, 9.4.2 |
| Content/Resources/ | The code prescriptions | 6.0 |
| BEM_utils/config.py | EnergyPlus version routing and the worker count | 9.3 |
| tests/test_neighbourhood_registry.py | A full simulation run of all 35 neighbourhoods, hours long. Not the install check | 9.4.1 |
| 0_BEM_Setup/SimResults_neighbourhoods/ | The raw run folders, one batch folder per run, with master_status.json. Not on GitHub, and the only copy | 9.4.4, 13.2 |
| outputs/ | Derived tables | 6.0 |
| docs_DONE/docs_LMN_web/ | The evidence behind every published number | 6.0 |
| regenerate_master_report.py, recover_master_report_from_csvs.py | Rebuild a batch summary after an interrupted run | 9.4.4 |

## Appendix B, status vocabulary

This section defines two distinct vocabularies used throughout this guide and the action plan: 1. Task status vocabulary: Describes the execution state of handover tasks, procedures and checklist items. 2. Number maturity vocabulary: Describes the scientific confidence and source backing of energy figures and quantities displayed in the tool.

### Task status vocabulary

These four terms govern every task, procedure and checklist status in this guide and in the action plan. No other status terms may be used for tasks.

| Status | Meaning | Symbol |
| --- | --- | --- |
| Completed | Done and verified | ✅ |
| In progress | Started, not finished, and it is in our hands | 🟨 |
| Waiting for CHV | Cannot proceed without Dr. Hachem-Vermette (e.g. Group D items, open method questions) | 🟦 |
| To be tested | Written and believed correct, never exercised by anyone but the author (Appendix C rows 5 to 8) | ⚠ |

### Number maturity vocabulary

Dr. Hachem-Vermette fixed these four terms and no others may be used to describe the maturity of an energy number or modelled quantity displayed in the interface.

| Term | Meaning |
| --- | --- |
| Simulation-backed | From an actual simulation output, for that exact case |
| Preliminary | From an implemented calculation whose method is not yet validated (e.g. Layer 3 EV/V2G, Layer 4 LPV) |
| In development | Present in the interface, not selectable |
| Not modelled yet | No model and no data. Shown only as a future possibility |

One further term exists for a case that is modelled but whose result is not publishable: Not currently available. It is deliberately kept separate from the four terms above.

## Appendix C, acceptance criteria for this guide

Criterion 1 is quoted in the words it was set in, on 2026-08-10, when the document was still called a report. The document is the same one.

| # | Criterion | Status | Note |
| --- | --- | --- | --- |
| 1 | One consolidated report exists, separate from documentation.html | ✅ Completed | Consolidated into single 3-part guide at v1.0, restructured at v3.1 |
| 2 | Every displayed result has a complete row in the Results Data Map with no empty cells | ✅ Completed | Verified in Section 5 data map |
| 3 | The data pipeline is documented from simulation output to live website | ✅ Completed | Verified in Section 7 |
| 4 | The manual steps in that pipeline are written out explicitly | ✅ Completed | Verified in Section 7.1 |
| 5 | Someone other than the author can update one neighbourhood result from this guide alone | ⚠ To be tested | Closes in the Appendix G test, task 6 |
| 6 | Someone other than the author can add a new neighbourhood from this guide alone | ⚠ To be tested | Closes in the Appendix G test, task 5 |
| 7 | Someone other than the author can deploy an updated live version from this guide alone | ⚠ To be tested | Not covered by Appendix G (which carries no deployment task). Deployment is section 11.1; this criterion is tested by the walkthrough in section 14.4, T3, T10 and T11 (D4, Waiting for CHV) |
| 8 | Backup and recovery are documented and tested at least once | ⚠ To be tested | Closes in the Appendix G test, task 7 |
| 9 | The guide contains no passwords or tokens | ✅ Completed | Verified: clean of secrets, accounts listed by permission |
| 10 | CHV has owner level control of the repository hosting the live tool | ✅ Completed | github.com/CarolineHVermette/NEXA-Web configured at owner level |
| 11 | CHV has reviewed the guide at least three times | 🟦 Waiting for CHV | One version has been shared, v1.0 on 2026-08-24, and she has commented twice since, on 2026-08-27 and 2026-08-30. No further version has been shared, and three reviews are not yet on record. This one, v3.2, is the second to be sent |
| 12 | The embedded token is revoked and the remote is clean | 🟨 In progress | P0 security: remotes are clean; token revoke on GitHub is Koral’s personal action (A1, section 14.5) |
| 13 | The simulation project is in an RHLab controlled archive | 🟦 Waiting for CHV | Mirror on CHV account current (T12); long-term independent archive is joint item D5 |

Six criteria are Completed, four are To be tested (rows 5, 6, 7 and 8), two are Waiting for CHV (rows 11 and 13), and one is In progress (row 12); each is stated where it belongs in the body rather than only here. Of the four to be tested, three are covered by the independent handover test in Appendix G, rows 5, 6 and 8. Row 7 is not, and is tested instead by the deployment walkthrough of section 14.4.

## Appendix D, revision history

This guide replaces the three documents that came before it. Until version 0.8 the handover set was three separate documents: a non technical summary, a technical orientation and this reference. They were written on different days for different readers, and the split followed the order they were written in rather than any reader’s need. They became one document in three parts at version 1.0, and the two shorter documents are archived unchanged in previous/. Nothing in Part III was cut in that merge. All fifteen sections and the three appendices were carried over whole; what was shortened is Parts I and II, which repeated them.

| Version | Date | What changed | Shared with CHV |
| --- | --- | --- | --- |
| 0.1 | 2026-08-10 | Skeleton, sections 1 and 2 | ⬜ |
| 0.6 | 2026-08-14 | All fifteen sections filled. Written as one pass rather than the planned five, at the request of the supervisor meeting | ⬜ |
| 0.7 | 2026-08-24 | Brought up to the published release. The PV efficiency corrected to the value that actually produced the results, the climate coverage reduced to the five published arms, the publish procedure corrected, the verification count updated, and the eight defects of the live check added to section 12 | ⬜ |
| 0.8 | 2026-08-24 | The simulation sources located, in a second repository with a remote on CHV’s own account. Sections 6.0, 13.2, 13.5 and 14 corrected. The eight defects closed, seven of eight fixed and verified | ⬜ |
| 1.0 | 2026-08-24 | The three reports merged into this one. Parts I and II added from Reports 1 and 2, shortened to what they did not duplicate. Six figures corrected against the published site rather than against the previous draft: the cache stamp, the external library count, the model-viewer size, the mirror state, the remote URLs and the fonts. Two items closed the same afternoon, the mirror push and the clean remotes, and one new defect opened and fixed, DBG-048 | ✅ 2026-08-24 |
| 2.0 | 2026-08-27 | Read this first added, at the front, in plain language: what the tool is, what is done, what is pending, and the ten questions that are for CHV. Nothing else changed. Every section, table and appendix is byte for byte the version 1.0 text. Written because the covering email of 24 August was not clear and CHV said so | ⬜ |
| 2.1 | 2026-08-27 | Two closures recorded in the opening, and nothing else touched. Stage 9 item 7 closed: ninety-four old notes, mockups, duplicate start pages and backup pages removed from the published site, kept offline. Stage 11 prepared as far as it can go without CHV. The pending list falls from six items to five | ⬜ |
| 2.2 | 2026-08-27 | The plain-language opening removed. What is done, what is still pending and the ten questions were a second copy of the action plan and are now only in the action plan. Sections 1 to 15 and the three appendices are untouched, and remain the version 1.0 text | ⬜ |
| 3.0 | 2026-08-27 | Renamed a guide. Handover and Maintenance Report becomes Handover and Maintenance Guide, and the document refers to itself as a guide throughout. No content changed: sections 1 to 15 and the three appendices are still the version 1.0 text | ⬜ |
| 3.1 | 2026-08-27 | Restructured as a guide. The revision history, the common change procedures and the record of the live site check moved out of the body into appendices D, E and F, so that the body carries only what a maintainer needs while working. Every heading rewritten as a plain title. No technical content changed: sections 1 to 15 and appendices A to C are the version 1.0 text | ⬜ |
| 3.1, addendum | 2026-08-27 | Section 8 completed against CHV’s request of 2026-08-27 that the guide state, step by step, how new information is added to the tool. Section 8 now opens with a table routing each kind of change to its procedure and its files, and two procedures were added: 8.8, updating text, tables and figures shown in the interface, and 8.9, adding a layer or a module and connecting another tool. Nothing else changed | ⬜ |
| 3.2 | 2026-08-31 | Handover closeout revision. Responded to Dr. Hachem-Vermette’s two emails of 2026-08-30. Standardised four-state task status vocabulary applied throughout (Completed, In progress, Waiting for CHV, To be tested). Group D items marked Waiting for CHV. Added Section 6.12 (simulation file naming), Section 8.10 (interface design), Section 15.0 (configuration vs simulation assumptions), and Appendix G (independent handover test). Later the same day, the Layer 3 source publication was identified and cited: Energy Reports 14 (2025) 4492 to 4507, section 2.4, equations 12 to 18, now recorded in sections 6.8, 15.3 and the new section 15.7, source publications. Rebuilt HTML and DOCX outputs via build.sh | ⬜ |
| 3.3 | 2026-09-09 | WP7/WP8 of the 2026-09-09 closeout plan (A24, A25). Resolved the archive placeholder in section 13.5 with the true open state (A5 to A8). Rebuilt section 12 into CHV’s four classes: real open defects, accepted V1 limitations, future research items, user choices, folding in the section 15.3 duplicates (flags f1, f2, the V2G unit defect). Fixed the dangling §10.3 cross-reference (now §10.2, where the line-ending baseline is explained). Renamed the product from LMN to N-LENS throughout the running text, title block and glossary; code tokens (LMN_CONFIG), file and folder names, the LMN sixteen-scenario factorial, repository/URL strings, and dated quoted historical text are unchanged, since they are real identifiers. File renamed to N-LENS_Handover-and-Maintenance-Guide_v3.3.md. A23, the full real-browser QA pass, had not been run at the time of this row, so this was a restructuring of the existing record rather than a new QA pass. A23 was run later the same day; see the 3.3 second addendum below and Appendix F2 | ⬜ |
| 3.3, addendum | 2026-09-09 | WP6 of the same plan (A12, A15, A22, A27, A28), and three defects found by running what this guide tells a new HQP to run. Six hand-maintained blocks moved out of the generated js/data.js into js/config.js, which closes the documented chart-colour exception; the climate fallback is 16 sites in 8 files, not the 21 in 10 stated before, measured; the verification suites’ line-ending assertions were rewritten to test for mixed endings, and the suite result is now 683 of 706, explained in section 10.2; a lab attribution block was added to both JSON exports; the cache stamp is ?v=25. New register entries DBG-049 and DBG-050, both fixed: the data pipeline could only run on one machine, and the two verification scripts named in nine places in this guide had never run at all. DBG-051 is open and recorded. A23, the full real-browser QA pass, had still not been run at the time of this row; it was run in the pass recorded immediately below | ⬜ |
| 3.3, second addendum | 2026-09-09 | WP7 of the same plan (A23). The full real-browser QA pass was repeated against the current state and this guide now carries it as Appendix F2. A real Chrome driven over the DevTools protocol against a served copy of the working tree, every clause of CHV’s wording: 385 baseline combinations across eleven envelope keys and all 35 neighbourhoods; 350 rungs of the efficiency ladder driven by clicking, on both arms of NECB Zone 6, against the 5 rungs of 2026-08-24; all 35 three-dimensional models; the eleven-step flow with a reload at every step and a back walk home; all 41 withheld pairs; the export against the screen, 4 of 4 metric rows and 19 of 19 assumption pairs; the print stylesheet; the console on every page; and the cache stamp, ?v=25, on all 15 pages and 65 references. Three new register entries, all open: DBG-052, DBG-053 and DBG-054. The first two are not reachable in the published flow. DBG-054 is P1 and is: the Thermal Load card is enabled everywhere and changes no number, because no cell in js/data.js carries the IAL column both resolvers ask for, 0 of 595, while the page still states that the measure was applied and directly simulated. No published number is wrong; a statement about one is. Section 12 introduction rewritten to say the classes are now confirmed by a fresh pass rather than built from the record alone | ⬜ |
| 3.3, third addendum | 2026-09-09 | A15 closed and the last two reachable defects of the A23 pass repaired. The 16 silent climate defaults are removed from 8 files, so no page defaults to a climate the user did not choose; DBG-052 is closed by vendoring Chart.js 4.4.3 into vendor/chartjs/, no CDN call added; DBG-053 by a keydown branch beside the existing click branch, so the seven card information icons open on Enter and Space; and DBG-051 fell out of the same edit. The cache stamp is now ?v=26 on all 15 pages and 66 references, and the verification suites read 685 of 706, two better, because the mixed line ending pair now passes. Sections 10.2, 12.1, Part II’s constraints and verification summaries and Appendix F2 are corrected; the dated appendix text itself is left as written. DBG-054 is untouched and is the one open question for CHV | ⬜ |
| 3.3, fourth addendum | 2026-09-10 | Appendix A extended into a directory glossary, at Dr. Hachem-Vermette request of the same day, so that a reader can find a file or a folder from one table. The existing fifteen terms become A.1 Terms, unchanged, and A.2 Folders and files is new: every path at the top of the repository, what is in it, and the section that describes it. It also records that docs_implementation/documentation-revisions/ and docs_methodology/, both named in section 4.6, are in .gitignore and are therefore not on GitHub. Section 12.0, question 2, is now answered, at the meeting of the same day: 1 TB for each project at a ceiling of CAD 300 to 400, on a portable SSD that Mac and Windows both read and write, the earlier 4 TB and 8 TB recommendation withdrawn. Section 12.0, question 3, is now answered, on Koral’s decision of the same day: Comparison Mode goes offline rather than to review, is kept in the repository so that it can come back, and its two entry points are commented out rather than deleted, which moved the cache stamp to ?v=27 on all fifteen pages. No other content changed | ⬜ |
| 3.3, fifth addendum | 2026-09-10 | The Thermal Load data was imported, so DBG-054 is half closed, and Comparison Mode was taken offline at the page itself. The simulations the IAL column needed were commissioned and finished the same day in the simulation repository: 210 runs, all 35 neighbourhoods across the six NECB climate zones, zero EnergyPlus Fatals. They were imported into js/data.js as an IAL block on each of those 210 cells, total, the six end uses and pv, taken at the one decimal the file already uses, and every one was read back out of the loaded file and checked against the source CSV, 8 fields on each of 210 rows and 0 mismatches, no all-zero row and no negative residual. Both resolvers now return IAL there rather than falling back. DBG-054 is half closed and not closed: the campaign covered the standard archetypes, so the high performance building still has no IAL column and the card still falls back there without saying so, 175 of the 350 combinations a visitor can reach. Sections 12, 12.0 question 1, the 12.1 row and Appendix F2 record that split. Separately, comparison.html answered a typed address with the working tool although both its entry points were commented out, so initComparisonPage now reads LMN_CONFIG.comparisonMode.published first and prints an offline notice instead; nothing was deleted and setting the flag to true runs the page as before. The two js changes moved the cache stamp to ?v=29 on all fifteen pages and 66 references. Both changes were published the same day, commit 5a956fb, pushed to the site repository and to the mirror on CHV’s account, and verified against the deployed server: 30 cases in a real headless Chrome, 30 passing, no console error and no failed request | ⬜ |
| 3.4 | 2026-09-11 | Renamed the tool from N-LENS to NEXA, on Dr. Hachem-Vermette’s email of the same day: “NEXA, Neighbourhood Energy eXploration & Analysis, Version 1 · Research Preview.” The name was applied in the running text, the title block, the glossary and the Group D items of this guide, in the website’s fifteen pages and three scripts, and in the repository README.md and the HQP trial package. The tagline is unchanged. As in the 3.3 rename, code tokens (LMN_CONFIG), repository and URL strings, and dated quoted historical text are unchanged, since they are real identifiers, and so is the revision row of 3.3 above, which records the previous name. The single source of the name remains LMN_CONFIG.releaseName and LMN_CONFIG.productName in js/config.js; no page writes it as a literal. The two js edits moved the cache stamp to ?v=30 on all fifteen pages and 66 references. Nothing was committed, pushed or published: the rename is local, version 3.3 remains the document submitted on 2026-09-11, and this version is the one prepared for the next submission | ⬜ |
| 3.4, addendum | 2026-09-22 | The live site moved to its permanent address, https://carolinehvermette.github.io/NEXA-Web/, served from github.com/CarolineHVermette/NEXA-Web, the destination repository. Header, sections I, 3, 11.1, 13.1, 14.2, 14.4 (T4, T7, T8), A.2 and Appendix C row 10 updated; D1 and D2 completed | ⬜ |
| 3.4, second addendum | 2026-09-24 | Findings of the HQP trial. Section 6.13 states that the index paths point into idf_reader; section 8.5 names both files a new technology option needs, js/config.js and layer2_energy_selection.html, step by step; section 9.4.4 says what the run folder is and where; section 10.1 says where to type the command and how to stop the server. New section 14.6, handover materials, and Appendix A.3, the files of idf_reader; the A.2 note on untracked folders corrected | ⬜ |
| 3.4, third addendum | 2026-09-25 | V1 wrap-up (WP6). Earlier the same day: DBG-055 (the V2G discharge unit defect) recorded as closed in sections 12.1 and 15.3, and section 5 gained an IAL row (the High-Performance restriction, DBG-054). This addendum: new Appendix G2, HQP trial findings and solutions, condensing feedback log items 1 to 16 to one table. Section 8.5 rewritten again: the WP4b fix made js/config.js the only file a new generation technology needs, since Layer 2 cards are now built by renderGenerationCards() from LMN_CONFIG.selectionLabels.generation, so the layer2_energy_selection.html step is removed. Sections 4.5, 7, 7.1, 8.1 and 9.4.5 rewritten for the WP4 pipeline fix: one DATA_VERSION constant in convert_master_csv.py, currently 2026-09-10, naming the dated Templates/ folder that carries the post-dfix national master and the IAL combined CSV; the legacy 2026-07-19/ CSV read only for US_ASHRAE/ASHRAE rows; stale CAN_MTL rows in the post-dfix master skipped; all four pipeline scripts confirmed runnable from any working directory | ⬜ |
| 3.4, fourth addendum | 2026-10-07 | New Appendix H, trial troubleshooting, at Dr. Hachem-Vermette’s request of 2026-10-06: the questions and issues of the HQP trial grouped into four areas, each with its explanation or solution, the text of the one-page document sent to her the same day. Nothing else changed | ⬜ |
| 3.4, fifth addendum | 2026-10-07 | Zhineng He’s completed trial. Section 9.4.1 step 4 corrected: tests/test_neighbourhood_registry.py is a full 35-neighbourhood simulation run, not an install check, and a one-line registry import replaces it. The simulation repository named by its current address, NEXA-desktop, in sections 6.0, 9.4.1, 13.2 and A.3. Appendix G2 rows Z1 to Z5, from his returned package. Appendix A.2: docs_methodology/ now tracked, with this guide in docs_methodology/HandoverDocument/ | ⬜ |

### Items awaiting joint completion with CHV (Group D)

The following six items from Dr. Hachem-Vermette’s email of 2026-08-30 are parked to be completed jointly with her following her upcoming travel. Note that other specific items across the guide (such as Layer 3 provenance in §6.8/§15.3 and solar reconciliation in §12.1) are also marked Waiting for CHV.

| ID | Item (CHV 2026-08-30) | Where tracked | Status | Date of request |
| --- | --- | --- | --- | --- |
| D1 | Finalise the live NEXA V1 deployment from the CHV or RHLab controlled repository | §11.1, §14.4 (T7) | ✅ Completed 2026-09-22 | 2026-08-30 (joint item, after CHV’s trips) |
| D2 | Update the remaining repository and live site links | §14.4 (T8) | ✅ Completed 2026-09-22 | 2026-08-30 (joint item, after CHV’s trips) |
| D3 | Establish the final NEXA V1 release or tag | §11.5 | 🟦 Waiting for CHV | 2026-08-30 (joint item, after CHV’s trips) |
| D4 | Go through repository access, deployment and recovery together until she can manage independently (also closes Appendix C row 7) | §14.4 (T3, T10, T11) | 🟦 Waiting for CHV | 2026-08-30 (joint item, after CHV’s trips) |
| D5 | Finalise the long term RHLab controlled archive and backup | §13.5, Appendix C (row 13) | 🟦 Waiting for CHV | 2026-08-30 (joint item, after CHV’s trips) |
| D6 | Koral’s access and role after the handover | §14.3, §14.4 (T9) | 🟦 Waiting for CHV | 2026-08-30 (joint item, after CHV’s trips) |

## Appendix E, common change procedures

The common changes a maintainer makes most often. Each names the section of Part III where the procedure is stated in full, and that full statement is the one to follow.

Change a label, a caption or a display assumption. Edit js/config.js. Nothing else. If the value you want to change appears literally in a page or a script, that is a defect: move it to config.js and have the page read it. Three separate defects in this round were the same value written in three files that then drifted apart. If you are changing a simulation assumption (an input to EnergyPlus), follow sections 8.7 and 15.0 instead, which require upstream re-simulation and pipeline re-import. If you are changing a post-processing assumption (the EV chain, the LPV chain, a solar efficiency), js/config.js holds only the caption: the change is made in the calculation source and the result regenerated, section 8.7.

Change the interface design. Adjust :root design tokens in css/styles.css for colours and typography, or edit buildSidebar() in js/sidebar.js for navigation layout. Remember that chart colours reside in ENERGY_COLORS in js/config.js. Bump ?v= on affected pages. Full map in Part III, section 8.10.

Update a neighbourhood’s results. Place the new campaign output under Templates/<date>/, run convert_master_csv.py then patch_data_js.py, then check_keys.py and test_data_flow.py, and check the affected neighbourhood on the Layer 2 pages and on the final summary, which read the same cell by different routes. Never hand edit a value in js/data.js to correct it; the next import overwrites it and nothing records that it was ever changed. Step by step, with the failure modes, in Part III, section 8.1.

Publish. The procedure is Part III, section 11, and it must be read there rather than summarised here, because two of its steps were wrong in an earlier draft in a way that would have cost a new developer real damage. The two things never to forget:

- Never git add -A. Stage with -u or by name, and read git status before committing.

- origin has two push URLs. A bare git push writes to both the site repository and the mirror. Push by explicit URL when you mean one of them.

## Appendix F, live site check of 2026-08-24

The tool was published on 2026-08-24 and checked the same day against the deployed server, first by two external language models and then by a real browser driven through the whole flow. One blocking defect was found and fixed the same day: the final results page offered a button to a page that is not published, so every click on it returned a 404. It was invisible in the repository, where that page exists on disk, and appeared only against the server. None of what the check then found was blocking.

Nine defects were found and all nine are closed. Eight are DBG-040 to DBG-047 in the register, seven of them fixed on the spot and the eighth, DBG-044, closed later the same day when the owner decided to vendor the 3D viewer library. The ninth, DBG-048, was found afterwards and is the reason this table has ten rows: it was not part of the browser pass at all, but surfaced when the question was asked what the last external host actually was. All fixes are verified in a browser, and the vendoring was verified against the server.

| ID | Where | Problem | Priority | State on 2026-08-24 |
| --- | --- | --- | --- | --- |
| STG-01 | Layer 2 PV page, and the scenario summary | The PV generation intensity was still the headline number, at 32px and with no label, although its results row had been replaced by total PV array area on CHV’s instruction. It was on two pages, not one: the same bar stood on the summary, which is the page a reviewer prints | P1 | ✅ Fixed. The bar is removed from both pages, and the function that drew it is deleted rather than left dead. It was not re pointed at another quantity, because a 0 to 100 kWh/m2 scale cannot be made to read square metres or MWh/yr without inventing a range. The intensity is still read, since the total in MWh/yr and the ratio of performance are computed from it, and it is still named in Assumptions and Model Information, which is where CHV asked the parameters to live |
| STG-02 | Landing page | “Get Started” was fully enabled with no pathway chosen and did nothing when clicked: no navigation, no message, no error. CHV had written the standard herself, on the button beside it: “It used to accept a click and do nothing, which reads as a choice” | P1 | ✅ Fixed. It now ships disabled with aria-disabled, prints the reason under it, “Choose a pathway above to continue”, and turns on the moment a pathway is chosen. A second gate inside the click handler means the disabled attribute cannot be cleared in the browser inspector to get past it, which is the pattern the climate page already used |
| STG-03 | Final summary, print | The printed scenario never named the tool. It carried the model and data version, but the string LMN appeared nowhere on paper | P2 | ✅ Fixed. The printed page now opens “LMN V1, the Layered Modular Neighbourhood tool. Version 1.0.0, last updated 2026-08-24.” The line is drawn on paper only, and its text comes from LMN_CONFIG, so it cannot drift from the version stamp |
| STG-04 | Final summary, print | The lab attribution and logo did not print, because .site-footer was hidden by the print stylesheet along with the navigation | P2 | ✅ Fixed. The footer prints, smaller, at the foot of the last page: the lab, the director and the logo. The navigation and the sidebar are still dropped, which is what that rule was for |
| STG-05a | Site wide | via.placeholder.com was requested as an image fallback and failed outright; the service is dead. Six references in published code | P2 | ✅ Fixed. Every reference is gone. An image that will not load is hidden, which is what the landing page already did. No page contacts that host any more |
| STG-05b | 3dviewer.html | ajax.googleapis.com serves the model-viewer library, so all 35 models depend on a Google CDN and none draws on a restricted or offline network | P2 | ✅ Closed 2026-08-24, on the owner’s decision. Version 4.2.0 is vendored to vendor/model-viewer/ and the one script tag now points at it. The library is 1,041,839 bytes, not the third of a megabyte estimated when the defect was written — the estimate was ours and it was wrong by a factor of three. Verified on the server: the vendored file returns 200 at that exact size and 3dviewer.html names no external host |
| STG-06 | Layer 3 selection | Four greyed cards gave no reason, while V2G Stations beside them said “Requires EV”. A reader could not tell “not built yet” from “not available until you choose something else” | P2 | ✅ Fixed. All four now carry NOT MODELLED YET, one of the four status words, and the greyed treatment every other unbuilt option uses. Every disabled card in the tool now states a reason |
| STG-07 | Layer 4 selection | The same status text was drawn in capitals on four cards and in title case on three. The source text was identical on all seven; the cause was two stylesheet rules of equal weight and opposite casing | P2 | ✅ Fixed. The status badge now states its own casing at a specificity the card rules cannot beat. One status word, one rendering, on every card on the site |
| STG-08 | Every page | /favicon.ico returned 404 on every page, because no page declared an icon. The console logged an error on every visit, which is the first thing a technical reviewer sees | P2 | ✅ Fixed. All 14 pages declare the lab logo as the icon. Zero 404s across the whole flow |
| DBG-048 | css/styles.css, every page | A ninth, found after the other eight and only because the Google Fonts question was asked. The stylesheet carried one @import of fonts.googleapis.com at line 3258, after the first rule. CSS discards an @import that follows a rule, so Inter and Outfit had never loaded on any page and everything rendered in the sans-serif fallback. The same line was also the last external host the site called | P2 | ✅ Fixed 2026-08-24. Both faces vendored to vendor/fonts/, four variable woff2 files covering weights 100 to 900, latin and latin-ext, about 180 KB, SIL Open Font License 1.1. Four @font-face blocks now sit at the top of the file and the @import is gone. Moving the line up would have fixed the loading and kept the external call, which is why it was vendored instead |

What the same check confirmed, so that the list above is read in proportion. All 14 pages load cold with no session and no script error. The state survives a reload at every one of the ten steps of the flow. The browser back button walks correctly back to the landing page. All 26 disabled controls are unreachable by keyboard. All 166 images load. All 35 three dimensional models render, on real graphics hardware. The exported file agrees with the screen on 47 of 47 numbers.

How the fixes were checked. All seven were verified in a real browser, against a served copy of the corrected code, and then the whole flow was run again: the five efficiency rungs driven through the interface one at a time by clicking, all 14 pages opened cold, and the eight verification suites re run. 706 checks, 696 pass, which is exactly the count before the fixes, so nothing regressed. The ten failures are all the same line ending checks, which fail because Git is configured to convert line endings on checkout; they are recorded in section 10.2 and are not defects in the tool.

The efficiency ladder, driven one rung at a time. This was the last open coverage box of the live site check, and it now passes on every count: the tool’s own resolver landed on the intended rung 5 times out of 5, the number on screen equalled js/data.js 5 times out of 5, CHV’s scenario name was printed for 5 of 5, and the ladder falls in the right direction throughout, 119.4, 112.8, 100.0, 86.7, 64.2 kWh/m2 per year.

## Appendix F2, live site check of 2026-09-09

A23 of the closeout checklist asked for the whole live site check to be run again against the corrected state, and not to rely on the pass of 2026-08-24 recorded in Appendix F. It was run on 2026-09-09 in a real Chrome driven over the DevTools protocol against a served copy of the working tree: real navigations, real mouse events at real coordinates, real key events, with the console, every network request and every dialog captured per page. The full record is docs_implementation/documentation-revisions/Results/RESULT-16_A23_Live-Site-QA_2026-09-09.md.

What it covered, against CHV’s own wording.

|  | Clause | Result |
| --- | --- | --- |
| 1 | All 15 pages, cold, no session state | Zero script errors, zero failed requests, zero external hosts, every image drawn |
| 2 | Back, Next, reload, direct URL entry | Eleven steps clicked end to end, reloaded on arrival at each, then walked back to the landing page. Zero findings |
| 3 | All active climates and NUs, baseline | 385 combinations, eleven envelope keys against all 35 neighbourhoods. Every number on screen equals js/data.js |
| 4 | The upgraded paths | The five rung efficiency ladder driven by clicking, on all 35 neighbourhoods against both arms of NECB Zone 6, 350 rungs against the 5 of 2026-08-24. The four heat pump rungs matched the stored total on 280 of 280, and the ladder never rises as a measure is added. The fifth rung is a defect, DBG-054 below |
| 5 | Disabled invalid combinations | All 41 withheld pairs draw their caution notice rather than a blank or a wrong number |
| 6 | Icons, images, 3D | All 35 models load and draw, checked on model-viewer’s own loaded property. 35 codes, 35 files, no gap either way. The error path prints a named message rather than an empty frame |
| 7 | Labels, units, status | Every greyed option card states its reason; no disabled control on any of the three selection pages takes tab focus |
| 8 | Print and export | The exported .json agrees with the screen on all 4 metric rows and all 19 assumption pairs; the print stylesheet drops the navigation and prints the table, the assumptions and the footer, with no horizontal overflow |
| 9 | Browser console | Clean on every page reached in the published flow |
| 10 | Cache and version stamps | ?v=25, 65 references, all 15 pages, nothing on an older stamp |

Three defects were found, DBG-052, DBG-053 and DBG-054, and all three were open and recorded in section 12.1 when this appendix was written.

Later the same day, 2026-09-09. DBG-052 and DBG-053 are closed, and DBG-051 with them. The paragraphs below are left as they were written, because they are the record of what the pass found. What clause 10 records as ?v=25 on 65 references is now ?v=26 on 66, the extra reference being the vendored charting library that closed DBG-052. DBG-054 is unchanged and still open.

2026-09-10, DBG-054 is half closed. The paragraph below is left as it was written, and one sentence of it has stopped being true. The missing simulations were commissioned and finished that day and their results were imported: 210 runs, 35 neighbourhoods across the six NECB climate zones, zero EnergyPlus Fatals, imported into js/data.js as an IAL block on each of those 210 cells and checked back against the source CSV field by field, 8 fields on each of 210 rows and 0 mismatches. So IAL is no longer present on 0 of 595 cells; it is present on 210, and both resolvers return it rather than falling back. What the paragraph still describes correctly is the high performance building, which the campaign did not cover. Measured over what a visitor can reach, the five published climates on both arms: 350 combinations, 175 answer with the simulated ideal load and 175 still fall back silently, all of them on high-performance-z4 to -z7b. The stamp that clause 10 records as ?v=25 stood at ?v=27 before this import and at ?v=29 after it and the Comparison Mode change of the same day.

Published and checked on the server the same day, commit 5a956fb. Pushed to both remotes, the site repository and the mirror on CHV’s account, and GitHub Pages rebuilt in about 45 seconds. The check was run against the deployed site and not the working copy, the same method as the pass above: a real headless Chrome over the DevTools protocol, 30 cases, 30 passing, no console error and no failed request on any page reached. 25 of 25 on the standard arm changed when the card was clicked and equalled the stored ideal load total to the decimal, across all five published climates and five neighbourhood types including the data centre. 5 of 5 on the high performance arm did not change, and the assumptions box there still prints “Measures applied: Thermal Load” beside the HPerf scenario paragraph, which is the sentence this appendix describes and the reason the entry is half closed and not closed.

DBG-052 is on comparison.html, which is gated and unpublished under A27: it loads a charting library that is not in the repository, so both of its charts are silently blank. DBG-053 is the seven card information icons, which take keyboard focus and cannot be opened from the keyboard because they are spans with a click handler rather than buttons. Neither is reachable by a user of the published flow.

DBG-054 is reachable, and it is the one finding of this pass that needs a decision. The Thermal Load card in Layer 2 is offered on every neighbourhood and every climate and changes no number. Both resolvers ask for an IAL column, js/energy.js line 566 and js/config.js line 1707, and no cell in js/data.js carries one: 17 envelope keys, 595 neighbourhood cells, IAL present on 0 of them. Both fall back to the baseline without saying so. Measured on 70 of 70 rows, the intensity shown after clicking Thermal Load equals the stored DEFAULT total, which is the same number the page shows with no card clicked, and the seven end use slices are identical to the decimal. What moves is the wording: the assumptions box prints “Measures applied: Thermal Load” while the scenario line still reads the baseline paragraph and “Where this number comes from” still reads “Directly simulated”. Nothing published is a wrong number. The value on screen is a real simulated baseline value. What is wrong is a statement about it. The column exists upstream, in LMN-web-full-results/CAN_MTL.csv, ASHRAE.csv and CAN_CLG.csv, so the way out is either a data import under the same traceability gate as every other column, or withdrawing the card until that import is done. It is not left enabled and inert.

One method note for whoever repeats this. The harness is disposable and was written in a session scratchpad; rebuilding it is about an hour, and the three things that cost time are worth carrying forward. The site sets scroll-behavior: smooth, so a rectangle read straight after scrollIntoView is the old one and every synthetic click lands in the wrong place unless the scroll is {behavior: 'instant'}. A JavaScript dialog freezes the renderer and every Runtime.evaluate with it, so Page.javascriptDialogOpening has to be handled or the run hangs. And Input.dispatchKeyEvent with keyDown alone does not make a button fire its click: Enter has to be sent as rawKeyDown, then char with text: "\r", then keyUp, or every keyboard test reports a false negative.

## Appendix G, independent handover test

This protocol provides an independent verification test for new researchers (Shiv and Furqan) to validate that the NEXA-web tool can be maintained, updated, and operated using this guide alone with minimal assistance from the author.

Deployment is not part of this test. The seven tasks are the ones Dr. Hachem-Vermette named, and none of them is a deployment. Publishing an update is section 11.1, and it is covered by the walkthrough in section 14.4, T3, T10 and T11, not here. Appendix C row 7 says the same.

### Environment

Before starting, ensure the working environment satisfies the prerequisites documented in section 9 (Dependencies). Only standard tools are required (Python 3.x, modern web browser, Git, and Node.js for verification scripts). Do not install external frameworks or libraries outside those listed in section 9.

### Test rules and scoring

- Self-reliance: The evaluator works exclusively from this guide and repository files. Minimal author assistance is permitted.

- Every question is a finding: Any question the evaluator must ask the author to complete a task indicates a documentation gap. Record each instance in the findings log against the section that failed to answer it.

- Scoring scale:

  - Pass: Task completed successfully using the guide alone with no questions asked.

  - Pass with questions: Task completed, but required questions or clarification from the author (logged as findings).

  - Fail: Task could not be completed, produced errors, or broke existing functionality.

Task 5 and task 7 are destructive in shape. Both run on a clone or on a branch. Never on main, and never on the live site.

### Test tasks

| # | Task | Guide section | Pass criterion | Result |
| --- | --- | --- | --- | --- |
| 1 | Run NEXA-web locally | 10.1 | The site serves and every page loads with no console error. |  |
| 2 | Locate a neighbourhood and its associated simulation and IDF files | 6, 6.12 | Given one NU code, they name the IDF, the weather file and the output folder. |  |
| 3 | Trace one displayed result back to its source | 5, 6.0 | Given one number on screen, they reach the CSV row and the campaign that produced it. |  |
| 4 | Make a small interface modification | 8.10 | An accent colour, a heading face or one label changes and shows locally. |  |
| 5 | Add or duplicate a trial neighbourhood or test case | 8.2 | A duplicated NU appears in the selection with its own data, without breaking the other 35. |  |
| 6 | Test one result update | 8.1 | One value changes through the pipeline and shows on the page, not by editing js/data.js. |  |
| 7 | Test backup and recovery | 13.3, 13.4 | The site is restored from a clone, and the mirror is restored from, not only pushed to. |  |

### Findings log

Record every question asked, ambiguity encountered, or documentation fix required:

| # | Guide section | Question asked or gap observed | Required documentation fix |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

## Appendix G2, HQP trial findings and solutions

The independent handover trial of Appendix G ran through September 2026, with Shivram Gopala Krishnan and Furqan Wali as the two HQPs, and Zhineng He as a third from 2026-09-21 to 2026-10-07 (rows Z1 to Z5). The table below condenses the sixteen findings and questions they raised into one row each, finding and final solution. It is a summary, not the record: the full account, including root cause analysis, workarounds tried along the way and message references, is docs_implementation/documentation-revisions/Shivram-Furqan/Feedback/FEEDBACK_LOG.md.

| # | Date | Task or section | Finding | Solution | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | 2026-09-17 | Setup, clean clone | docs_implementation/documentation-revisions was missing from Furqan’s clone, gitignored by default | .gitignore given directory exceptions to track the three files the trial needs | Closed |
| 2 | 2026-09-17 | Task 2 Step 8, traceability | Templates/2026-07-21/ Montreal Baseline values (178.8) did not match js/data.js (179.2), a real gap left by the dfix baseline correction applied to js/data.js but not to Templates/ | Pipeline fix of 2026-09-25: convert_master_csv.py now reads the post-dfix master from Templates/2026-09-10/ (the DATA_VERSION constant), so a pipeline rerun reproduces the published 179.2 with 0 diff | Closed |
| 3 | 2026-09-18 | Task 3 Steps 4-5, pipeline update | Master CSV edits did not reach js/data.js; patch_data_js.py’s path bug was confirmed already fixed, the real cause was convert_master_csv.py reading the whole legacy CSV with no standard filter and overwriting Montreal | Fixed 2026-09-25: the legacy 2026-07-19/ CSV is now read for US_ASHRAE/ASHRAE rows only, so it cannot clobber the DATA_VERSION master’s NECB zones | Closed |
| 4 | 2026-09-18 | Task 3, Montreal workaround | Commenting out the legacy CSV read raised an IndentationError, and dropping it entirely failed test_data_flow.py with 0 ASHRAE NUs matched, since the legacy CSV was the only ASHRAE source | Same fix as item 3: the legacy CSV stays in the read order, restricted to ASHRAE rows, so Montreal and ASHRAE both resolve correctly with no manual workaround | Closed |
| 5 | 2026-09-21 | Task 4 Step 9, serve locally | The View Neighbourhoods button stayed grey after adding a trial neighbourhood; not reproduced, probably a script error in the edited js/data.js or js/config.js stopping updateAvailableOptions() before it finished | Check the browser console for the first script error after editing js/data.js or js/config.js; no code change applies | Answered |
| 6 | 2026-09-21 | Task 3 and Task 5, reload | CSV and token changes were not visible after re-hosting, probably the browser’s cache of the ?v= stamped css/js files | Guide section 10.3 (private window or disable cache reload) | Answered |
| 7 | 2026-09-21 | Task 6, guide 8.5 | Unclear whether a new technology needs js/config.js or layer2_energy_selection.html edited; confirmed both were needed under the procedure at the time | Fixed 2026-09-25: generation cards are now rendered by renderGenerationCards() from LMN_CONFIG.selectionLabels.generation; only js/config.js is edited, no HTML edit | Closed |
| 8 | 2026-09-21 | Checklist | Checklist sections B and C could not be found among the materials sent to the HQPs | CHV’s blank checklist sent (NEXA_Handover-Closeout-and-HQP-Trial-Checklist.docx), listed in guide 14.6 and trial sheet 01 | Answered |
| 9 | 2026-09-22 | Task 2 Part B, trace one number | Furqan traced 253.2, the IAL (ideal air loads) value, as the Baseline EUI of RS-I3 Montreal, instead of the DEFAULT value 179.2 | Guided to redo Part B against the DEFAULT level; not a code issue | Answered |
| 10 | 2026-09-22 | Task 2 Step 8 | Master CSV “approximately matches with unusual roundoff”, actually the same pre/post-dfix gap as item 2 (178.8 vs 179.2), not a rounding difference | Same pipeline fix as item 2: Templates/2026-09-10/ carries the post-dfix master, so the CSV and js/data.js agree | Closed |
| 11 | 2026-09-22 | Task 1 and Task 2 Step 3 | Guide should add a step to stop the local server with Ctrl+C, and note that the index idf_path points into the idf_reader clone, not NEXA-web | Guide sections 10.1 (stop the server) and 6.13 (index paths are idf_reader paths) written | Answered |
| 12 | 2026-09-24 | Guide 8.5, Task 6 | Section 8.5 did not say which file or files to change | Rewritten again on 2026-09-25 so the procedure is config only: icon, one js/config.js entry, cache stamp bump, check; no HTML edit any more | Closed |
| 13 | 2026-09-16 | Task 8 Step 4, guide 9.4.4 | “Run folder” was read as a folder literally named Run | Guide 9.4.4 and Task 8 Step 4 define it as the batch folder under idf_reader/0_BEM_Setup/SimResults_neighbourhoods/ | Closed |
| 14 | 2026-09-24 | Checklist item 8 | Checklist cited in the materials but never received by the HQPs | Sent, same checklist as item 8; listed in guide 14.6 | Answered |
| 15 | 2026-09-24 | Handover, email | Furqan suggested listing the files and documents of both NEXA-web and idf_reader in the handover section | Guide section 14.6 (handover materials) and Appendix A.3 (idf_reader files) written | Closed |
| 16 | 2026-09-24 | Item 11 follow-up (Ctrl+C, 6.13) | Promised guide lines for stopping the server and the idf_path note | Guide sections 10.1 and 6.13 written | Closed |
| Z1 | 2026-10-02 | Task 6, trial sheet 6 | Zhineng He: sheet 6 still asked for a static HTML card in layer2_energy_selection.html, and its example lacked unavailable: true | Sheet 6 rewritten 2026-10-07 to the config-only procedure of section 8.5 | Closed |
| Z2 | 2026-10-02 | Task 8, clone | idf_reader answered “Repository not found”: it was renamed NEXA-desktop and is private | Collaborator invitation; sections 6.0, 9.4.1, 13.2, A.3 and trial sheets 01 and 8 name NEXA-desktop | Closed |
| Z3 | 2026-10-07 | Task 8 Step 1, guide 9.4.1 | The prerequisite tests/test_neighbourhood_registry.py could not be confirmed as passing after repeated attempts | It is not a quick test: it simulates all 35 neighbourhoods at import, for hours, and rewrites a tracked input model. Section 9.4.1 and sheet 8 now use a one-line registry import as the install check | Closed |
| Z4 | 2026-10-07 | Task 2, Simulation Model Index | The index lists RS-I3 component_buildings as Restaurant Fast Food and Primary School 50pct, while the site shows 28 attached houses, a primary school, a small retail and a quick service restaurant | Real and known when the package was written (trial sheet 02). The index is generated in NEXA-desktop and is not corrected in this repository | Open |
| Z5 | 2026-10-07 | Tasks 2 and 3, reference values | Sheets 2 and 3 still gave the pre-correction 178.8 kWh/m2 and one expected null lookup; the site shows 179.2 and test_data_flow.py reports 0 | Sheet 02 reference values moved to the Templates/2026-09-10/ master, sheet 3 expects 0 null lookups (DBG-027 filled 2026-09-15) | Closed |

## Appendix H, trial troubleshooting

Questions and issues raised while testing NEXA in the independent trial, grouped into four areas, each with its explanation or solution. It was sent to Dr. Hachem-Vermette on 2026-10-06 as a separate one-page document and added here at her request. The item by item record is Appendix G2.

| Area | Issues raised | Explanation and solution |
| --- | --- | --- |
| 1. Setup and Access | Missing documentation folder after cloning, no step to stop the server, empty 3D viewer, simulation repository “not found”. | Trial files are now in the repository. The guide explains Ctrl+C, and the package opens the viewer through “View 3D”. The simulation repository was renamed NEXA-desktop and is private: access by invitation. |
| 2. Data and Traceability | Master CSV and website values differed (178.8 vs 179.2 kWh/m²), Baseline traced to the wrong level, model paths outside NEXA-web, Montreal edits not reaching the site. | The old CSV predated the 15 September correction; the trial now uses the current folder. The script overwriting Montreal is fixed. The guide clarifies model paths and the Baseline entry. |
| 3. Website Edits | Changes hidden after reload, “View Neighbourhoods” stayed grey, colour and label edits not shown, unclear file for new technology options. | Browser cache: use a private window. Grey button: a typing error in an edited data file, shown in the console; both passed on retry. Colour and label steps corrected. Technology options now come from one config file (guide updated). |
| 4. Recovery and Handover Materials | Purpose of the rollback task unclear, “run folder” ambiguous, checklist not received, full handover file list requested. | Task 7 purpose added (undo with history kept, rebuild from the repository). Run folder defined. Checklist sent 24 September; guide 14.6 and Appendix A.3 list the handover files. |